AGENT UPLOAD - Copyright 2002 - Net-Tamer, INC. all rights reserved.

Net-Tamer, Inc.

P.O. Box 372310

Satellite Beach, FL 32937-0310

Phone: (321) 777-4249

Fax: (321) 779-2530

E-mail: nettamer@nettamer.net

----INTRODUCTION-------------------

Agent is a stand-alone automated FTP file upload and download program.

It works only with a comm port connection (usually a modem) to a PPP

interface to a TCP-IP network. In other words, it can be instructed

to dial a telephone number, log in to an internet provider, call an

FTP server and upload and/or download the files that it finds in the

FTP.UP and FTP.GET files. THESE ARE TEXT FILES THAT YOU, THE USER,

MUST MAKE TO TELL THE PROGRAM WHAT TO DO. There are many other features,

which are covered by this document, so please read carefully.

This is a demo program. It requires the user to press enter, after

the program starts, before it proceeds with dialing the telephone,

and doing it's business on the internet. It is intended only for

use for 30 days or less, so that someone can determine the program

suits their needs. It is anticiapted that most users of this program

would want it to be called from another program or a TSR timer. If

the press enter was not there, there would be no need to purchase

anything.

If you have need of a program like this, then you may purchase it

for $40.00 from Net-Tamer, Inc. (see details in the order.txt file).

The comercial version of this program is identical to this one,

EXCEPT that it has the User's name on it and there is no press enter

to continue pause.  This is accomplished by emailing the user a

registration code and not another copy of the program.

We also customize this software to suit corporate and governmental

users and have competively priced licensing arrangements. We do not

customize or make changes to the program for single users.

----RESTRICTIONS ON USE AND DISTRIBUTION-----

The files contained in this archive may be reproduced for distribution

on any shareware CD ROM, Diskette or other media, provided that the

files are distributed in the orginal archive and that the purchaser

of that CD ROM, diskette, or other media is not lead to believe that

they have purchased the rights to the program. All right's are retained

by Net-Tamer, Inc. and anyone wishing to purchase the rights to use

this product must do so through the company.

----SYSTEM REQUIREMENTS------

Agent.exe should run on any 8088/8086 compatible  PC (including

the orginal IBM PC) with a modem and 120k or so of free memory.

It should run from a floppy, ram drive, and/or hard drive, if

configured correctly. DOS 3.2 or higher, or a system supporting

a "DOS shell" is required. No other software (other than the

OS) is required, but the program may be called by another program

via a "shell and execute" routine, if desired. Embeded systems

usage may require changes in the agent.exe code. Let me know, if

you have a need for this.

----Y2K---------------

All Net-Tamer products use 4 digit years in any spot where a date is

used. Users should be aware that the date the program uses is reported

to the program by a call to DOS. Early versions of DOS did not support

dates past 12-31-1999 and many older computer BIOS's have the same

problem. Because of this, the program has a special Y2K help feature.

If the program gets a report from DOS of a year that is less than 1999,

the program assumes that 20 years should be added to the date and that

the day of the week should also be corrected. In other words, if your

computer won't handle a date past the end of 1999, then on Jan. 1, 2000,

reset your date to Jan. 1, 1980 and the program will correct accordingly.

Since Jan. 1, 1980 was a Tuesday (something all XT users know), the

program will add 20 years and then correct the day of the week to

Saturday, which is correct for the year 2000.

Please understand. This only affects dates used by the program and

does not affect any other program or your operating system. The easiest

solution for all programs is to change DOS versions. This is not possible

on some embeded products or other closed systems, however, which is why

the date correction feature has been added to Net-Tamer products.

-------PPP Dial Up Capasity-----------

The program assumes dynamic IP addressing, unless you use the

keyword STATIC in a login script (see the section on the script

language.) The program will login to a service which uses

CHAP(the RFC compliant kind) or PAP, without a script.

The program handles all login negotiations.

----SETUP - THE AGENT.INI FILE---------

You will need the following information:

1) The telephone number of an internet service provider.

2) A username and password for that service.

3) If you expect to identify the FTP site to be contacted as

a URL, then you will need to provide a DNS server address. You

do not need a DNS server address, if the site will always be

given as an IP address, or the service supports assignment of the

DNS address on login (most do). In other words, SITE1=ftp.mysite.com ,

will require a DNS server address to resolve, but SITE1=206.100.113.2

won't.

4) You will need a login name and password for that FTP site.

If not give, then these default to your ISP username and password.

If you are using an anonymous FTP site, then entries should be

anonymous as the username and your email address as the password. i.e.

FUSER=anonymous

FPASS=myaccount@some-isp.com

For your personal web site, the usual requirement is the login username

and password.

5) You will need to know your modem comm port. If you have a non-standard

comm port using a special IRQ, this can be entered in the INI file.

6) You will need to know what you want to upload and/or download in

advance. If you are uploading file with the same name as ones that

already exist on the FTP site in that directory, then what happens

depends on how the service has that site configured. i.e. the other

file may or may not be over written, and you may or may not get an

error code.

Also read the comments in the sample AGENT.INI file.

None of the Keywords are case sensitive

PORT=

Will accept a number from 1 to 4

or a hex address, irq

Irq's 2 to 15 are okay in most circumstamces. Some WIN 95 users,

may have to change IRQ's for the modem in My Compter/Defaults to

7 or less. When in doubt check the settings for another comm program.

BAUD=

Valid 300, 1200, 2400, 4800, 9600, 19200, 38800, 57600, 115200

Most 14.4 modems should be set to 57600 and never below 19200.

Most 28.8 modems will need 115200.

NUMBER1=,NUMBER2=,NUMBER3=,NUMBER4=

Telephone number to call

The # used deterimines dialing order. if a number is busy. Only #1

Is required to be used. A number is required, but it may be "none".

For example, NUMBER1=NONE , would tell the program not to dial, but

it would still try to init a modem before starting the connection,

if an init string had been given. If there were no init string and

the number is NONE, then the program would wait one second and then

start the script file, if given, or go straight into PPP, if a script

file is not given.

SCRIPT1=,SCRIPT2=,SCRIPT3=,SCRIPT4=

The name of a script file to use, if the service does not use CHAP or

PAP user verification. Default is no script. NO PATH is specified. A

script file is assumed to be in the same directory as the exe file.

THE SCRIPT NUMBER MUST MATCH THE TELEPHONE NUMBER, ie. SCRIPT1 is for

NUMBER1, etc.

DIALSTART=

Dialing string start usually ATDT (The default)

INIT=

Modem Init String, Usually ATZ will get it.

DNS1=,DNS2=

IF a DNS address is NOT given, then the program will attempt to

use IPCP options 129 and 131 (0x81 and 0x83). Not all servers

support these options. If the FTP address is not given and a URL

is used, then DNS1= must be set, OR the service must support the

IPCP options in question. DNS1= is primary. DNS2=is secondary.

HANGUP=

Usually ATH0^M, the default. +++ is assumed.

GIVEUP=

Most attempts to dial with out terminating. Default is 2000

DDTIME=

The number of seconds to delay between redials for a busy signal. The

default is no delay.

USERNAME=

The login username for PPP PAP or CHAP. Also assumed to be the

username for the FTP Site unless FUSER is defined.

PASSWORD=

Password for PAP or CHAP and the FTP Site unless FPASS is defined.

SITE1=,SITE2=

FTP URL or Address. You may define a second, if the first is unavailable.

If you do not include a port number, then FTP is assumed to be to on PORT 21

for the command channel and PORT 20 for the data channel. This is per the

RFC's. You may use a different port, however, by includeing a colon and the

port number at the end of the URL. MOST SITES USE PORT 21!!!! Do not include

a port number, unless you know that port 21 is incorrect.

Examples:

SITE1=ftp.somesite.com or SITE1=125.201.15.2 are assumed to be port 21 for

commands and port 20 for data.

SITE1=ftp.somesite.com:9007 or SITE1=125.201.15.2:9007 would be assumed to

be port 9007 for commands and port 9006 for data. The data port is always

the command port minus one.

FUSER=

Override for FTP Username.

FPASS=

Override for FTP Password

FDIR1=

Upload FTP directory

FDIR2= is assumed to go with SITE2=, etc.

This should be the complete CWD command as the program needs to send it

to the server.

*NOTE: Unix name systems are case sensitive.

HISTORY=

File to write files sent,errors, etc.

Include Drive and path, if not the same as AGENT.EXE

DOWNDIR=

The DOWNload directory. The program will check for a file named

FTP.GET, which is(are) the file(s) to retrieve.

IF THIS FILE Exists, then program will download the files

listed and put them in this directory.

WINDOWS=

If the program is being run under Windows 95 (or 98), setting this

to any value tells the program to use the Windows Timer API.

DNLOADFIRST=

If the program is to download files, before it uploads files,

then setting this to any value.

MTU=

The program defaults to a maximum transmission unit of 1500, as per

the RFC's. Sometimes, a particularly bad phone line, bad UART, or

bad connecting modem will make the program unable to complete a send.

(There is seldom a problem in receiving.) If this occurs, then set

the MTU down. MTU may be any value between 128 and 1500. The most

common error that needs this correction is "FTP DATA TIME OUT".

PASSIVE=

The program defaults to a PASV open of an FTP server. If this creates

a problem, then set PASSIVE=0 in the INI file.

----DEFAULTS--

DialStart=ATDT

GiveUp = 2000

FUser=

Defaults to Username

FPASS=

Defaults to Password

INIT=ATZ

PORT=1

BAUD=19200

HangUp=ATH0^M

History=AGENT.HST

DOWNDIR defaults to current directory.

------COMMAND LINE SWITCHES-------

/CAP instructs the program to write all packets - in and out- to

a file named agent.cap. This is an append file and is useful to

me in diagnosising problems users might have. Use to anyone else

is questionable, unless they can read the hex dump.

/IG instructs the program to ignore the FTP.GET file for this run only.

/IU instructs the program to ignore the FTP.UP file for this run only.

(Obviously, don't use both at once;)

/S Sets showall to on for this run only. This is the same as setting

SHOWALL=1 in the ini file.

-----Setting Up the FTP.UP and FTP.GET files----------

The files FTP.GET and FTP.UP are assumed to be in the agent.exe

startup directory. These are text files which specify files to retreive

and files to send.

FTP.UP format    (The file listing which files to send.)

This may list as many files as you wish with a pass. Wild Cards

are okay. The command |ASC tells the program to upload as

an ascii file. The default file type is IMAGE (binary).

Notice that |ASC must be uppercase and have at least one space in from of

it.

The |DELE command must be after any |ASC command and before any |RNTO

command. It tells the program to delete the file from the user's drive,

IF upload was successful.

(examples)

c:\agent\upload\agent.exe

c:\agent\upload\file1.txt |ASC |DELE

C:\FRED\FUN.EXE |DELE |RNTO AllInGoodFun.EXE

C:\agent\xfile.html |DELE

c:\agent\upload\file1.txt |ASC |DELE |RNTO file1.html

The |RNTO command must be the last thing on the file name line.

It tells the program to upload the file to the name following

RENTO on the FTP server. |RNTO cannot currently be used with a wildcard

file name. Please note that unix filenames are case sensitive and that

the program will use the exact file name specified.

FTP.GET format (The file listing which files to retreive.)

Use unix path names, if a path is required. *.*  or *.ext are the only

wild cards allowed.

USE |ASC, if not a binary file.

/u2/home/nettamer/*.*

*.123 |ASC

/mydir/fred.zip

USE |DELE if files are to be deleted on the server once

download is complete. (examples)

/u2/home/nettamer/*.*  |DELE

*.123 |ASC |DELE

/mydir/fred.zip |DELE

----HISTORY FILE MESSAGES AND DOS TERMINATION ERRORS-----

IF the DOS termination error is below 100, then it is an internal

error message. Those codes are below.

ERROR Nothing To Do! (Error 100)

There were no FTP.GET or FTP.UP files.

ERROR No Phone Number! (Error 101)

There was no telephone number to dial. If the program is on a

leased line, then use PHONE1=NONE in setup.

Max Dials Exceeded. (ERROR 102)

(Maximum Dial Attempts was reached with not connection.)

ERROR No Site Give. (Error 103)

(No Site Was Listed In the INI File.)

ERROR Cannot Find Site.  (Error 104)

(DNS look up bombed. On the Address(es) given.)

ERROR Cannot Open Socket. (Error 105)

(The FTP site(s) was (were) down.)

ERROR Call Rejected By Ftp Server. (Error 106)

(The FTP site rejected the call.)

ERROR Login Not Allowed or Bad Password. (Error 107)

(The username/Password was not accepted by FTP Server or login was rejected.)

ERROR Lost Contact. (Error 108)

(The server went offline before the file could be sent and verified.

A completed send is possible, but unlikely.)

Okay 192.0.1.91

or

Okay ftp.myservice.com

(The upload was completed and verified to the server listed.)

User Terminated. (Error 109)

(Someone hit F10.)

SCRIPT ERROR (Error 110)

The login script was bad.

Not Found: Sitename (Error 111)

The DNS server could not resolve the service name

Attempted Send Filename.ext (Error 112)

Generic upload failure

Cannot Download FileName.Ext (Error 113)

Generic Download failure

Echo Loop. Check Login Script. (Error 114)

The service likely has login prompts. A script will be needed.

Service Requested Hang Up. (Error 115)

The ISP asked the program to log off, and it complied.

PPP Error. (Error 116)

Generic PPP problem.

PPP CHAP Failure. (Error 117)

The CHAP Athentication Failed. This could be because the username

and password were wrong, or because the service uses Microsoft CHAP

instead of RFC compliant CHAP.

Password Reject (Error 118)

PAP failed.

----INTERNAL ERROR NUMBERS-----

5 Illegal function call. This is a VERY generic error.

Check filenames and numbers for size, etc.

6 Overflow -A number exceeded 2,000,000,000.

7 or 9 Out of memory

11 Division By ZERO.

14 Out of string space (same as 7 or 9)

17 Cannot continue

A Dos error. Leave the program, re-boot and try again.

24 Device time out - Modem, or drive is not working.

25 Device fault          ""

51 Internal Error

Same as 17.

52 Bad file name

53 File NOT found

55 File already open

57 Device I/O error

Same as 24.

58 File already exists

59 Bad record- Usually a zero byte file.

61 Disk full - Delete something.

62 Input past end of file- Same as 59

63 Bad file number - Duplicate file name given.

64 Bad file name - Invalid dos file name

67 To many files- Up the number FILES= in config.sys

68 Device unavailable

see 24

71 Disk NOT ready

72 Disk media error

75 File access error

71-75 are all user drive errors. Run scandisk.

76 Path NOT found.

Invalid path used in the ini file.

--------------LOGIN SCRIPT LANGUAGE-----------------------

Please note that this is only how a user can script the "dialer"

part of the program for services with login prompts. In other

words, systems who do not use PAP or CHAP authentication.

There is no need in this language for WAIT CONNECT. The script

only starts after the modem has connected to another modem.

Keep the prompts that the program waits for short. As a general

rule, 4 characters is plenty. So use:

WAIT word:

- instead of -

WAIT Password:

The dialing scripts use two "macros" these are Password$ and

Username$. These send those pieces of information to

a service. Do not change those words, Agent knows what

the formation is and keys on this are to do certain things!

Use:

Blast PUSERNAME$

Instead of:

Blast USERNAME$

if your service requires a P in front of your username. Most don't.

Some services require something AFTER the Username, just edit the

script this way:

Blast Username$XXX

The program will Replace Username$ with your username, add the

XXX stuff and a <13>. Do not add <13> to Username$ and Password$

in a script. The program adds these for you.

If you need to include an Ascii control character in a command,

just enter it like this <27>. (That would be the escape character.)

Agent DOES NOT ALLOW SPACES ON THE RIGHT OR LEFT HAND SIDE OF A

WAIT COMMAND. USE <32> to indicate a space. Spaces in the middle

of a statement are recognized. i.e. <32>NOW IS THE TIME<32> would

check for the text and a space on either side.

Below is a modified script for an access site. Yours may or may not

function in the same way. This is just a sample.

WAIT gin:-OR-#600

DELAY 10

BLAST Username$

WAIT word:

DELAY 10

BLAST Password$

WAIT ocol:

DELAY 10

Blast PPP<13>

DELAY 10

PPP

NOTE:

The last PPP in the file is an internal command to the program to go into

PPP mode. Nothing in the script file is executed past this point.

DELAY #-

This command pauses program execution for X tenths seconds

(decimals are not accepted). The incoming characters are still

checked.

example: DELAY 15 (wait 1.5 seconds)

WAIT TEXT-OR-TEXT-OR-TEXT -

OR

WAIT TEXT-OR-TEXT-OR-#12345

This command tells Agent to check the incoming characters from

service until the TEXT information is received. The -OR- is optional.

If you specify -OR- (max two) the program will resume

execution when it receives the characters matching any command

specified by TEXT. The maximum length of TEXT is 40 characters. You

may also say wait BILL-OR-#120. The number preceded by the #

character indicates the maximum length of time to wait. The number

is 1/10ths of a second.

If the time is up, before the program reaches the text, then the

script is abandoned. If a wait statement is not successful and you

have not specified a time, Agent abandons the script in 7 seconds

on a *.LOG file. <32> below represents a space character.

Example: WAIT MAIL><32>:-or-TO:<32><32>

Add /NOERROR, if you do not wish the script to end, if the text is

not found in the specified time.

BLAST TEXT<13>

This command tells the program to send the characters in text. If

the <13> command is included, the program sends ASCII 13 or enter.

Example:  Blast READ WAITING NS<13>

IF TEXT -

or

IF NOT TEXT

END IF -

Tells Agent to compare the current state of incoming data for

service with TEXT. If the TEXT condition is true, then Agent

proceeds otherwise, it skips to the end of the END IF.  The NOT

statement REVERSES this. The statements are executed only if the

TEXT is not the situation.

STATIC ###.###.###.### -

STATIC 206.14.56.13 in the script at any time before the PPP

command, would tell the program to assume that as your static IP address.

This is seldom needed. The most likely use would be a direct

serial port connection to network, and not over a modem.

PPP -

This ends the script file and tells the program to go to PPP

mode. YOU MUST INCLUDE THIS COMMAND AT THE END OF THE SCRIPT!

USERNAME$ and PASSWORD$ - These are instructions to the program

to insert your Username or Password in this spot in the script.

DO NOT INCLUDE A <13> with these commands. For example:

BLAST USERNAME$

Will send your username and a <13>.

You may also use these commands, even if other information is

needed in the login. For example, CRIS.COM uses USERNAME$PPP

to tell the service that you want a PPP connect. Netcom.com uses

#USERNAME$ for the same reason. Again, no <13> is required.

Sample scripts:

-----DEMON.CO.UK-------

wait login:

BLAST Username$

wait Password:

BLAST Password$

wait Protocol:

BLAST PPP,nolqm,MRU=1500<13>

wait HELLO

PPP

-----------------

----CIS1-----

DELAY 30

Blast <3>

WAIT ame:

BLAST CIS<13>

WAIT ID:

BLAST Username$/go:pppconnect

WAIT word:

BLAST Password$

DELAY 50

PPP

----------------

----CIS2------

DELAY 30

Blast <13>

WAIT ID:

BLAST Username$/go:pppconnect

WAIT word:

BLAST Password$

DELAY 50

PPP

-----------------

----CRIS----

WAIT user:-OR-#600

BLAST Username$@PPP

WAIT word:

DELAY 10

BLAST Password$

PPP

-----------------

----EROLS--------

BLAST <13>

WAIT Login:

BLAST Username$

WAIT Password:

DELAY 10

BLAST Password$

WAIT PPP

PPP

----------------

----PACIFIER-----

WAIT  host:

BLAST ppp<13>

WAIT login:<32>

BLAST Username$

WAIT word:<32>

DELAY 10

BLAST Password$

PPP

-----------------

----PIPELINE-----

DELAY 40

BLAST @

ECHO @

DELAY 3

BLAST D

ECHO D

DELAY 3

BLAST <13>

WAIT in:

BLAST DELPHI<13>

WAIT Username:<32>

BLAST Username$

WAIT Password:<32>

BLAST Password$

Wait Hello

DELAY 5

PPP

-------------------

----AGATE-----

DELAY 10

BLAST <13>

DELAY 10

WAIT ogin:<32>-OR-#600

DELAY 10

BLAST Username$

WAIT  word:<32>

DELAY 10

BLAST Password$

WAIT  beginning....

PPP

------------------