tm

PC-STRAN    v5.0x  Users Guide

tm

PC-STRAN

A 1-,2- or 3-Dimensional

STRuctural ANalysis Program

For the IBM Personal Computer

(C)Copyright 1984 - 1994

by Joe Murphy,Ph.D.,P.E.

Version 5.0x

Joe Murphy

P.O. Box 56164

Madison,  WI  53705

(608) 274-1946

tm

PC-STRAN   is a registered trademark of Joe Murphy.

IBM is a registered trademark of

International Business Machines Corp.

Microsoft is a registered trademark of

Microsoft Corporation.

Hercules is a registered trademark of

Hercules Computer Technology, Inc.

Page 1 of 58

tm

PC-STRAN    v5.0x  Users Guide

Table of Contents

Disclaimer...................................  3

Acknowledgements.............................  3

General Information..........................  4

System Requirements..........................  5

Some Definitions.............................  6

Installation.................................  7

Getting Started..............................  9

Example Problem..............................  9

CFGSTRAN..................................... 11

Viruses, Corrupt files and File Integrity.... 11

Tips & Techniques............................ 11

Selection Techniques......................... 13

Numbers and Letters...................... 13

Scrolling................................ 13

Mouse.................................... 13

\Paths\...................................... 14

Hercules MonoGraphics........................ 14

Maximums and Mass Storage.................... 15

NON-normal Program Termination............... 15

Use of EXTENDED Memory....................... 16

Information about PC-STRAN files............. 17

INSTALL files............................ 17

DOC files................................ 17

DATA files............................... 18

PROGRAM files............................ 18

TEMPORARY files.......................... 20

EXTENDED Memory files........................ 20

Previous Versions Compatibility.............. 21

Program details.............................. 22

PC-PROP.................................. 22

Estimating Torsion Constants........... 23

PC-MESH.................................. 25

Computing Roll Angle................... 28

Tieing Nodes Together.................. 29

Advanced Feature....................... 30

PC-LOAD.................................. 31

PC-STRAN (General) PC-SOLVE (Particular). 33

Limitations............................ 34

PC-VIEW.................................. 34

PC-RPT................................... 35

Load Factor Analysis................... 35

Graphics Hardcopy............................ 36

Run-Time Errors.............................. 36

References................................... 37

PC-STRAN Version / Enhancements.............. 39

Obtaining the Latest Version................. 41

Some Questions answered...................... 42

Page 2 of 58

tm

PC-STRAN    v5.0x  Users Guide

Permission to COPY........................... 43

User-Supported Software...................... 44

Shareware.................................... 46

Documenting Problems......................... 47

Invoice...................................... 47

8087 Version................................. 48

Command Line Switches/n...................... 48

Video Override Switch........................ 49

Mouse Switch................................. 49

Test Machines................................ 50

Index........................................ 51

Disclaimer

----------

Use  of  this  program  acknowledges  this   disclaimer   of

warranty:

This program is supplied as-is.  The  Author  disclaims  all

warranties,   expressed   or   implied,  including,  without

limitation, the warranties of merchantability and of fitness

of this program for any purpose.

Although every effort has been made to  ensure  both  theory

and  programming  logic  in  this  program  are correct,  no

warranty, either expressed or implied is made by the  Author

that the analyses obtained from its use will be correct.  In

no  event  will the Author be liable to you for any damages,

including any lost profits, lost savings or other incidental

or consequential damages  arising  out  of  the  use  of  or

inability  to  use this program, even if the Author has been

advised of the possibility of such damages, or for any claim

by any other party.

Acknowledgements

----------------

I gratefully acknowledge the following engineers

Catherine M. Marx, P.E.

Ron Wolfe, P.E.

Dave Gromala, P.E.

Ross M. Lanius, Jr., P.E.

They and users are entitled to credit for

features you like in PC-STRAN.

Page 3 of 58

tm

PC-STRAN    v5.0x  Users Guide

General Information

-------------------

PC-STRAN analyzes 2- or 3-Dimensional truss/frame structures

or 1-D continuous beams. The general procedure is to  recall

three disk files:

1) a member PROPerty file containing:

a) member type

b) material properties

i) density

ii) Young's modulus

iii) (torsional) shear modulus

c) geometric properties

i) cross sectional area

ii) 2 (bending) moments of inertia

iii) (rotational) torsion constant

2) a geometric 3 dimensional MESH file containing:

a) structure's node information

i) node number

ii) 3 coordinates

iii) 3 lateral restraints

iv) 3 rotational restraints

b) structure's member information

i) beginning & end node numbers (2ai)

ii) 2 member-joint fixity factors

iii) roll angle

iv) member type (1a)

3) a LOAD file with up to 8 load cases containing:

a) loaded node number (2ai)

b) loads at 3a) with:

i) 3 applied forces

ii) 3 applied moments .

c) loaded member node numbers (2bi)

d) linearly distributed loads at member nodes

PC-STRAN employs a matrix  method  of  structural  analysis,

specifically  the direct stiffness method of solution, which

is sometimes called the displacement method. See section  on

References.

Page 4 of 58

tm

PC-STRAN    v5.0x  Users Guide

A truss/frame is defined as a structure whose members can be

attached  to  joints  (or nodes) where only axial forces are

transmitted (a truss with frictionless  swivel  connections)

OR  where  bending and twisting moments are also transmitted

(a frame with rigid connections).  A truss/frame member then

can have one end of the member a swivel connection  and  the

other end of the SAME member a rigid connection!

Concentrated loads may  only  be  specified  at  nodes.   If

concentrated  loads  are  to be applied at the interior of a

member, insert a node at that point and  divide  the  member

into  two.   For  member  loads,  the  values  of the linear

distribution  are   specified   at   the   members'   nodes.

Displacements are calculated only at the nodes.  If you want

displacements  at  the  interior of a member, again insert a

node at that point.

For more information see Section on Program details.

System Requirements

-------------------

PC-STRAN requires:

1) structural engineering knowledge

(most IMPORTANT)

2) PC/MS DOS \Path\ knowledge

MD\    : make subdirectory command

CD\    : change subdirectory command

COPY   : copy file command

(quite important)

3) IBM PC or IBM XT or IBM AT with:

a) DOS 2.1 or higher

b) Memory of 448K for largest problem

(dependent on problem size)

c) CGA, EGA, VGA, HGC, or HGC+, MCGA,

80 column monitor

d) 2 disk drives (1 w/400K free)

(preferably 2 drives & RAM disk)

Program subdirectory (drive) w/500K free

Data subdirectory (drive) w/360K free

[Note: For 2 360K drives see Section on Installation]

Page 5 of 58

tm

PC-STRAN    v5.0x  Users Guide

Some Definitions

----------------

PC-MESH and PC-VIEW can view and rotate  the  3  dimensional

structure.  Also,  defining  the  coordinates  of  the nodes

require  defining  a  three   dimensional,   right   handed,

Cartesian  coordinate  system.   This will be referred to as

the GLOBAL system.  Below is a representation:

y

|

|

|      L

|     /|

|    / |

|   /  |

|  /   |

| /    |

|/     |

O------|------------x

/ \     |

/   \    |

/     \   |

/       \  |

/         \ |

/           \|

/             P

z

The x,y,z coordinate system has the +z axis pointing at you,

the +y axis pointing vertically up, and the +x axis pointing

to your right.  PC-MESH will  initially  (by  default)  view

this way - you looking in the -z direction.

LONG & LAT angles

If you are looking from point L towards the  origin  O,  the

angle  LO  makes  with  its projection PO on the xz plane is

defined the "LAT"itude angle, measured positive from the  xz

plane  toward  the  +y  axis.  The angle PO makes with the z

axis is defined the "LONG"itude angle measured positive from

the +z axis toward  the  +x  axis.   These  two  angles  are

similar to LONGitude and LATitude on a globe of the world.

Page 6 of 58

tm

PC-STRAN    v5.0x  Users Guide

Installation

------------

===========================================================

HARD DISK -

First Way - INSTALL.BAT --  Put  DISK2  in  drive  A,  type

"install" and answer the questions appropriately.

Second Way - Do-it-yourself -- Create a subdirectory,  call

it  \PCSTRAN\, on your hard disk.  Then create the following

subdirectories,   \PCSTRAN\PROGRAMS\,   \PCSTRAN\DOCS\   and

\PCSTRAN\DATA\.   Copy  all .EXE, .COM and .BAS files to the

PROGRAMS subdirectory.  Copy all .DOC and .BAT files to  the

DOCS  subdirectory  and  copy all .PRP, .MSH and .LDS to the

DATA subdirectory.

Change  to  the   PROGRAMS   subdirectory.    Run   CFGSTRAN

responding to the default paths that describe your system.

Copy  PC-STRAN.EXE  to be in your default path (usually your

hard disk root directory.)  This way you can  type  PC-STRAN

at the DOS prompt > no matter what subdirectory or drive you

are   in.   Run  EFILECHK.   You  should  get  IBM  extended

character 251 (square root mark) if all the .EXE  files  are

OK.   If  you get an IBM extended character 168 (upside down

question mark), call Joe Murphy immediately.

===========================================================

===========================================================

A 720K, or 1.2Mb DRIVE - Format  a  new  disk.   Create  the

following  subdirectories,  \PROGRAMS\,  \DOCS\  and \DATA\.

Copy  all  .EXE,  .COM  and  .BAS  files  to  the   PROGRAMS

subdirectory.   Copy  all  .DOC  and  .BAT files to the DOCS

subdirectory and copy all .PRP, .MSH and .LDS  to  the  DATA

subdirectory.

Change   to   the   PROGRAMS   subdirectory.   Run  CFGSTRAN

responding to the default paths that describe your system.

Copy  PC-STRAN.EXE  to  be  in  your  root  directory.   Run

EFILECHK.  You should get IBM extended character 251 (square

root  mark) if all the .EXE files are OK.  If you get an IBM

extended character 168 (upside down question mark), call Joe

Murphy immediately.

Change to your root directory.  Run PC-STRAN.

===========================================================

Page 7 of 58

tm

PC-STRAN    v5.0x  Users Guide

===========================================================

TWO 360K DRIVES - Format a new disk in drive  B.   Copy  all

the  files  from  DISK2  in  drive A to the disk in drive B.

Save and mark the disk "PROGRAMS".

Format a new disk in drive B.  Copy COMMAND.COM to the  disk

in drive B.  Copy all the files from DISK1 in drive A to the

disk in drive B.  Save and mark the disk "DOCSDATA".

Put the disk from drive B (i.e., DOCSDATA) in drive A.

Put your "PROGRAMS" disk in drive B.   Change  to  drive  B.

While in drive B run PC-STRAN.

===========================================================

Page 8 of 58

tm

PC-STRAN    v5.0x  Users Guide

Getting Started

---------------

First install your software.  See Section on Installation.

These steps specified will be for a two 360K disk system.

1. Boot up the computer.  Put your DOCSDATA disk in drive A.

Put your PROGRAMS disk in drive B.  Switch to drive B.

2. Type PC-STRAN at the prompt (e.g. B>pc-stran)

3. You will be queried for the Path and PROBLEM filename for

the DATA files, the Path for the PROGRAM files, the Path for

the STORAGE files, and the Path for the  WORK  files.   Give

appropriate   responses.    You   can  use  the  destructive

backspace key.

4. When in the PC-STRAN Main Menu, you can choose 1,2,3,4,5,

6, or 7 to transfer to PC-PROP, PC-MESH, PC-LOAD,  PC-SOLVE,

PC-VIEW, PC-RPT or restart PC-STRAN respectively.

5. Once you are done with PC-PROP, PC-MESH, PC-LOAD or

PC-VIEW, you can transfer to the PC-STRAN Main Menu.

6. When you  choose  PC-SOLVE,  option  4,  the  RESult  and

DEFormation files will be written using the Path and PROBLEM

filename  of the DATA files while the STORAGE and WORK files

will be written and deleted on their specified Paths.

7. To start out, work  through  the  example  problem  of  a

truss.

Example Problem  -- a 3-D truss with 6 nodes & 12 members

---------------

1. Boot up the computer.  Put your DOCSDATA disk in drive A.

Put your PROGRAMS disk in drive B.  Switch to drive B.  Type

PC-STRAN at the prompt (e.g. B>pc-stran)

Page 9 of 58

tm

PC-STRAN    v5.0x  Users Guide

2.  For  this  example  and  assuming you followed the above

installation you should type:

example1.(enter)

(enter)

(enter)

(enter)

where (enter) means pressing the Enter key.

3. At PC-STRAN Main Menu select option 1 -- transferring  to

PC-PROP

4.  At  PC-PROP  Main  Menu  select  option  "4)  Print  all

...records".  When queried where to print, select option "s)

Screen".

5. After viewing the records and returning to  PC-PROP  Main

Menu try editing a record.

6. To file the updated records select option  "9)  FILE  all

records...".

7. When you want to return to the main menu,  select  option

"0) END ... transfer...".

8. PC-MESH and PC-LOAD are  similar  to  PC-PROP.  Read  the

internal instructions for viewing the mesh in PC-MESH.

9. PC-SOLVE is fairly straightforward.  After  the  analysis

ends,  you  will return to the main menu.  You can return to

DOS or transfer to PC-VIEW to view the deflected  structure.

If  you  return  to  DOS  the DATA Directory should have the

following files on it. EXAMPLE1.PRP, .MSH, .LDS, .RES, .DEF.

10. To get a REPORT, either

1) select Main Menu Option 6, or

2) load BASICA (e.g. A>BASICA ). Then load PC-RPT  (e.g.

LOAD"a:\pc-rpt" ).  Answer A:\EXAMPLE1. to "Path and PROBLEM

filename  of DATA files", B:\ to "Path for PC-PAGE.EXE", and

A:\ to "Path of WORK files".  PC-RPT  adds  the  appropriate

filename extensions.

PC-RPT will prompt if you want to do a load factor analysis.

EXAMPLE1.RPT is an ASCII file that you can send to a printer

with the command (e.g. A>print a:\example1.rpt) or  view  on

screen with the command (e.g. A>more <a:\example1.rpt).

11. That's all there is to it.

Page 10 of 58

tm

PC-STRAN    v5.0x  Users Guide

CFGSTRAN

--------

CFGSTRAN.EXE can be run at any time to change  the  default

paths in PC-STRAN.EXE.  Just make sure that CFGSTRAN.EXE and

PC-STRAN.EXE  are in the same default directory when you run

CFGSTRAN.EXE.

Viruses, Corrupt files and File Integrity

-----------------------------------------

EFILECHK checks if even one byte has been  changed  in  the

.EXE   files   developed  and  distributed  by  the  author.

EFILECHK checks the .EXE files  in  the  default  directory.

You should get IBM extended character 251 (square root mark)

if  the  .EXE  file  is  OK.   If  you  get  an IBM extended

character 168 (upside down question mark), call  Joe  Murphy

immediately.

Tips & Techniques

-----------------

Keep programs separate from data and documentation.  If  you

use  a  hard disk, copy all .EXE, .COM and .BAS files into a

separate PROGRAM subdirectory.  Copy  .DOC  and  .BAT  files

into a separate DOCS subdirectory.  Copy .PRP, .MSH and .LDS

files  into  a separate DATA subdirectory.  For each problem

you could create a separate subdirectory.

Keep units CONSISTENT!  All forces MUST have the same units!

All length units MUST be the same!

If you have a RAM drive, designate it as the drive  for  the

WORK files!

File records often (select option 9) when using PC-PROP,

PC-MESH or PC-LOAD - you will not end the program  but  will

save  data  if  something  should happen.  Option 0 will NOT

file data.

Page 11 of 58

tm

PC-STRAN    v5.0x  Users Guide

Rotating the structure in PC-MESH should flag obvious errors

in geometry and in member end connectivity.

The default structure size in  ViewMesh of PC-MESH and

PC-VIEW allows the ENTIRE structure to be  viewed  from  any

angle.  If the structure appears small use the zoom feature.

The default node restraints in PC-MESH is  0  (0  movement).

The default member end connections in PC-MESH is 0 (0 fixity

factor).

When minimizing the node bandwidth (option m in PC-MESH) use

a target value  of  2.   After  repeated  passes,  note  the

minimum  bandwidth  that was attained during a pass.  Do NOT

resequence the nodes.  Choose option m again  but  now  with

the  minimum  bandwidth  (you know it was attainable) as the

target bandwidth.  Resequence the nodes.

The experienced user does not have to use  PC-PROP,  PC-MESH

or  PC-LOAD  to  create  or update the *.PRP, *.MSH or *.LDS

files.  Any ASCII word processor can  be  used.   Use  UPPER

CASE letters!  If in doubt about how the files should be set

up check the .PRP, .MSH and .LDS data files!

Page 12 of 58

tm

PC-STRAN    v5.0x  Users Guide

Selection Techniques

--------------------

There are three ways to make  selections  at  the  PC-STRAN

Main Menu and PC-PROP, PC-MESH, PC-LOAD, PC-VIEW Menus:

1) Number and Letter Keys

2) Scrolling and pressing the Enter Key

3) Using a Mouse

Numbers and Letters

-------------------

In the text menus, the user can press the number or  letter

in front of the close parenthesis ")" to enter that option.

Scrolling

---------

In the text menus, the user can use the PgUp and PgDn  keys

to  bounce  the  highlighted  bar  over  the desired option.

Pressing the Enter key selects the highlighted option.

Mouse

-----

In the text menus, moving the mouse up or  down  moves  the

highlighted bar.  Pressing the left mouse button selects the

highlighted option.

In the PC-PROP, PC-MESH, and PC-LOAD file editor, the mouse

movements and buttons correspond to the following keys:

right          - Tab

left           - Shift Tab

down           - PgDn

up             - PgUp

left button    - Enter

right button   - Esc

Page 13 of 58

tm

PC-STRAN    v5.0x  Users Guide

In  the  PC-MESH  and  PC-VIEW  graphics  mode,  the  mouse

movements and buttons correspond to the following keys:

no buttons pressed

right          - Right Arrow  => +latitude

left           - Left Arrow   => -latitude

down           - Down Arrow   => -longitude

up             - Up Arrow     => +longitude

left button pressed before movement

and released after movement

right          - "r"          => +Xshift

left           - "l"          => -Xshift

down           - "d"          => -Yshift

up             - "u"          => +Yshift

right button pressed before movement

and released after movement

right          - "b"          => bigger displacements

left           - "s"          => smaller displacements

down           - "o"          => zoom out

up             - "i"          => zoom in

\Paths\

-------

Beginning with Version  4.00,  PC-STRAN  accepts  Paths  to

DATA, PROGRAM, STORAGE and WORK files.

Hercules MonoGraphics

---------------------

Beginning with Version  4.00,  PC-STRAN  graphics  will

work  with  the  Hercules  Graphic Card and Hercules Graphic

Card Plus.   One has to install a Hercules DRIVER by  typing

MSHERC  at the DOS prompt before starting PC-STRAN.  See the

Information about the PC-STRAN files.

Page 14 of 58

tm

PC-STRAN    v5.0x  Users Guide

Maximums and Mass Storage

-------------------------

PC-STRAN has the following maximums:

Number of member types              =    64

Number of member types (MASTER).PRP =   512

Number of nodes (NN)                =   128

Number of members (NM)              =   256

Semibandwidth (IH)                  =   255

Number of degrees of freedom (DOF)  =   768

Number of load cases (LC)           =     8

Number of members with

linearly distributed loads         =   128

(per load case)

Number of records, K matrix (NR)    = 32727*

NR = IH * [(IH + 1)/2 + (DOF - IH)]

PC-STRAN needs the following mass storage:

On the WORK files (RAM) drive

ERASE-ME.TMP

On the STORAGE files drive, a maximum of 456K

ERASE-ME.ROT    1152 bytes * NM

ERASE-ME.MB#      72 bytes * NM * LC

ERASE-ME.ND#      24 bytes * NN * LC

*NOTE: Beginning with Version 5.00, PC-STRAN looks first  to

store  the  K  matrix  in  EXTENDED  memory.   The Number of

records is NOT limited to 32727 if there is enough  EXTENDED

memory.   If there is no EXTENDED memory or not enough, then

PC-STRAN will use conventional memory  (below  640K  limit).

In  conventional memory, the number of records IS limited to

32727.  See Section on Use of EXTENDED Memory and Section on

EXTENDED Memory Files.

NON-normal Program Termination

------------------------------

If you have a  non-normal  program  termination  (normal  is

option  0  in  PC-STRAN  to  DOS), please let me know if you

think it is in the  program.  Before  using  PC-STRAN  again

check for and ERASE all the ERASE-ME.* files.

Page 15 of 58

tm

PC-STRAN    v5.0x  Users Guide

Use of EXTENDED Memory

----------------------

To  work with EXTENDED memory use an extended memory manager

XMM such as HIMEM.SYS developed and distributed by Microsoft

Corporation.  An example CONFIG.SYS file is:

break = on

DEVICE=HIMEM.SYS

device=xmsdisk.sys 3328K

device=ansi.sys

files= 40

buffers= 40

The second device, XMSDISK.SYS, creates a 3328K "RAM"  drive

(electronic  or virtual drive) in EXTENDED memory.  The rest

of EXTENDED memory can be used by PC-STRAN!

; XMSDISK.SYS --- XMS-aware RAMdisk device driver

; Copyright (C) 1989 Ziff Davis Communications

; PC Magazine * Ray Duncan

;

; To install:   copy XMSDISK.SYS to the boot disk, add

;

;               DEVICE=XMSDISK.SYS  nnnK

;

;        to the CONFIG.SYS file.  This must follow

;        the DEVICE= line that loads the XMM (usually

;        HIMEM.SYS).  The parameter nnn is the desired

;        RAMdisk size in KB.  If nnn is missing or zero,

;        all available extended memory is used.

Page 16 of 58

tm

PC-STRAN    v5.0x  Users Guide

Information about PC-STRAN files

--------------------------------

This  information  is  provided  so  that  you'll  know  the

function of each file distributed to you or made by

PC-STRAN,  and  whether  or  not each file is optional.  The

following files have a '+' if provided on  the  distribution

diskettes and have a '*' if optional:

INSTALL files

-------------

+*  IFDIR.COM - A  stand-alone  file  used  by  INSTALL?.BAT

files to check for the existence of subdirectories.

+*  INSTALL?.BAT - Batch files to copy the appropriate files

into subdirectories \PROGRAMS\, \DATA\ and  \DOCS\.   Should

be used with a hard disk.

+*   PATHS.SCR  -   An  ASCII  listing  of  paths  used   by

INSTALL?.BAT to configure PC-STRAN with the correct paths to

programs and data.

DOC files

---------

+*  ADDED.DOC  -  If  this  file  is  present,  it  contains

additional documentation.

+*  DOC.BAT - The file is used to assist you in printing out

the  documentation.   It  is  also  not  needed  after   the

documentation has been printed.

+*  INVOICE.DOC - A one-page ASCII  text  file  form  to  be

filled out and sent along with registration fee.

+*  PC-STRAN.DOC - This is the main documentation file.   It

is not needed after the documentation has been printed.

+*  PROBLEM.DOC - A ASCII text file form to  be  filled  out

and  sent  along with any INPUT files,  if the author cannot

resolve the  problem  by  phone.   Also  use  the  form  for

documentation errors or enhancement suggestions.

+*   README1.DOC  -  A  file on steps to use PC-STRAN.  This

file is not needed after the documentation has been printed.

+*  README1.EXE - A program to display README1.DOC.

Page 17 of 58

tm

PC-STRAN    v5.0x  Users Guide

DATA files

----------

*  filename.DEF - file containing DEFlected  node  geometry

of  loaded  structure.  Together with filename.MSH,  PC-VIEW

can overlay loaded structure on top of non-loaded structure.

+*  filename.LDS - file containing LoaDS for  INput  in  PC-

STRAN. example.LDS are provided as optional examples.

+*  filename.MSH - file containing MeSH  geometry  -  INput.

example.MSH are provided as optional examples.

+*  filename.PRP - file containing member  PRoPerty  data  -

INput.   Maximum  number  of  entries is 64. example.PRP are

provided as optional examples.

+*  (MASTER).PRP - file containing member  PRoPerty  data  -

INput.   Maximum number of entries is 512.  If PC-SOLVE does

not find filename.PRP,  PC-SOLVE looks for the  member  type

in (MASTER).PRP.  (MASTER).PRP must contain the member types

in alphabetical order.

*   filename.RES  -  file  containing  PC-STRAN  RESults  -

OUTput.   This  file is INput to PC-RPT for writing "custom"

reports.

*  filename.RPT - file containing RePorT of analysis.

PC-RPT generates this as OUTput.

PROGRAM files

-------------

+*  CFGSTRAN.EXE - A stand-alone program to reconfigure  the

default \Paths\ in PC-STRAN.EXE.  See Section CFGSTRAN.

+*  EFILECHK.EXE - A stand-alone  program  that  checks  the

file   integrity  of  all  the  *.EXE  files  developed  and

distributed by the author.  See Section on Viruses,  Corrupt

Files and File Integrity.

+*   HIRESPIX.EXE  -  A   stand-alone,   terminate-and-stay-

resident  (TSR),  program  similar to IBM's GRAPHICS.COM for

Epson and compatible dot matrix printers.  Works  with  CGA,

EGA, VGA, MCGA, HGC and HGC+.

Page 18 of 58

tm

PC-STRAN    v5.0x  Users Guide

+*   MSHERC.COM - A stand-alone, terminate-and-stay-resident

(TSR), driver for monographics with Hercules  HGC  and  HGC+

cards.

+   PC-DIR6.EXE - A program called by PC-STRAN to  list  all

the  files  in  the \DATA\ subdirectory in 6 columns.  Files

are in CAPITAL letters and subdirectories are in lower  case

with a "*" marking them.

+   PC-LOAD.EXE - A child program to create or update a file

containing applied LOADs at the nodes of the structure. This

program is called from PC-STRAN.

+   PC-MESH.EXE - A child program to create or update a file

containing the unloaded structure geometry  or  MESH.   Also

the  program  is  for  VIEWING  the  structure  (rotating  &

zooming). This program is also called from PC-STRAN.

+   PC-PAGE.EXE - A program called by PC-RPT to paginate the

filename.RPT file.

+   PC-PROP.EXE - A child program to create or update a file

containing the PROPerties of the members of  the  structure.

This program is also called from PC-STRAN.

+   PC-SOLVE.EXE - A child program that SOLVES the

1-,2- or 3-Dimensional STRuctural ANalysis of the  structure

using, as input, files created by PC-PROP, PC-MESH, and

PC-LOAD.

+   PC-STRAN.EXE - A parent program that controls the  1-,2-

or 3-Dimensional STRuctural ANalysis of the structure. After

entering DOS to start, type in PC-STRAN (e.g. B>PC-STRAN)

See Section Command Line Switches.

+   PC-VIEW.EXE - A child program  for  VIEWing  the  loaded

structure   (dashed  lines)  over  the  original  structure.

Similar to PC-MESH.  Also for viewing graphs of member load,

shear,  moment,  slope  and  displacement  diagrams.    This

program is also called from PC-STRAN.

+   PC-RPT.BAS - A stand-alone source program  that  can  be

user   modified   for   custom  report  writing.   INput  is

filename.MSH and filename.RES and OUTput is filename.RPT.

+   PC-RPT.EXE - A child program for RePorT  writing.   This

program can be called from PC-STRAN.

Page 19 of 58

tm

PC-STRAN    v5.0x  Users Guide

+*   ROLLANGL.BAS  -  A  stand-alone  source   program   for

interactively  computing the roll angle of a member in three

dimensions.

+*  ROLLANGL.EXE - An executable ROLLANGL.BAS.

+*  UPDATE25.BAS - A stand-alone source  program  to  mm  to

modify  .LDS  files  created using version 2.50 to work with

following   versions.    See   Section   Previous   Versions

Compatibility.

+*  VIDFIX.EXE - A stand-alone,  terminate-and-stay-resident

(TSR),  patch for early-IBM BIOS.  If PC-STRAN seems to lose

its way and  you  have  an  early-IBM,   run  VIDFIX  before

running PC-STRAN.

TEMPORARY files

---------------

ERASE-ME.ROT - A temporary STORAGE file for  the  member

stiffness  matrices  and  local-to-global ROTation matrices.

It should be killed or erased by  PC-STRAN  upon  successful

termination.

ERASE-ME.ND#, .MB# - Temporary STORAGE  files  for  NoDe

and MemBer loads of load case #.  It is killed by

PC-STRAN.

ERASE-ME.TMP  -  A  TeMPorary  WORK  file  for  PC-STRAN

results.   The results are reformatted for filename.RES.  It

is killed by PC-STRAN.  Also TeMPorary WORK for PC-VIEW.  It

is erased by PC-VIEW.

EXTENDED Memory Files

---------------------

HIMEM.SYS - An extended  memory  manager  developed  and

distributed by Microsoft Corporation.  Allows PC-STRAN

to  utilize  extended  memory  to store the global stiffness

matrix K. See Section on Use of EXTENDED Memory.

XMSDISK.SYS - An extended memory  aware  RAMdisk  device

driver.  Use for tempory STORAGE and WORK files.  Saves wear

and  tear  on mechanical disk drives.  See Section on Use of

EXTENDED Memory.

Page 20 of 58

tm

PC-STRAN    v5.0x  Users Guide

Previous Versions Compatibility

-------------------------------

For Versions before 2.0

If you have made DATA files with a previous version of

PC-STRAN and you want to use those files with this  version,

you  must  RENAME  the  files  to have a common filename and

extensions PRP, MSH and LDS. Also insert a line (use  EDLIN)

at the very beginning of the LDS file with a 1 in the second

column.  (It signifies 1 load case.)

For Versions before 2.5

If you have made a .LDS file  compatible  with  Version  2.0

(see  above),  run  the  UPDATE25.BAS program to rewrite the

.LDS file!

Page 21 of 58

tm

PC-STRAN    v5.0x  Users Guide

Program details

---------------

PC-PROP

-------

PC-PROP  creates  a  catalog  of  member  properties.   The

created file is stored on disk and used as an input file for

PC-STRAN. This eliminates specifying member properties every

time  an  analysis  is  run.   Substituting  a  member in an

analysis requires changing only the member type field in PC-

MESH.   (MASTER).PRP  is  searched  for  if  there   is   no

filename.PRP.

PC-PROP can recall a member PROPerty  file  from  disk,  and

modify  member  records,  add records, delete records, print

all the records to disk, printer or screen, and/or file  the

updated records.

PC-PROP is menu driven with scroll record data entry.  There

are 8 fields in each member type record.  They are:

1) Member Type name - alphanumeric characters

(NOTE: Must not be NODE2NODE, See Section on

Tieing Nodes Together)

2) Density - force/length cubed

3) Young's Modulus - force/length squared

4) (torsional) Shear Modulus - force/length squared

5) Cross Sectional Area - length squared

6) (bending) Moment of Inertia about local y axis -

length fourthed

7) (bending) Moment of Inertia about local z axis -

length fourthed

8) (rotation) Torsion Constant about local x axis -

length fourthed

Fields 1,3,5 MUST be defined.  Fields 6,7 are necessary when

bending moments are  to  be  transmitted.   Fields  4,8  are

necessary when twisting moments are to be transmitted.

Page 22 of 58

tm

PC-STRAN    v5.0x  Users Guide

Estimating Torsion Constants

----------------------------

If field 8 is left blank it will be approximated by

PC-STRAN using information in fields 5,6,7 by:

4

A

TC = -----------------

2

(2*PI) (Iyy+Izz)          PI=3.141593

This approximation can be applied with an error of  no  more

than about 12%, even when the cross section is a triangle or

narrow  rectangle. The cross section must have no projecting

arms  or  reentrant  angles.   Also  this  approximation  is

useless  for  tubes  whether  they  are  closed or slit open

lengthwise.

To compute the approximate torsion constant for  thin-walled

closed  tubes  of  any cross-sectional shape and of constant

wall thickness, t, use the following formula:

2

4 a  t

TC =  --------

p

where a is the whole area bounded by the centerline  of  the

perimeter,  p, of the tube.  The area a is approximately the

average of the two areas enclosed  by  the  inside  and  the

outside  surfaces  of  the tube.  It is the area enclosed by

the centerline of the wall's contour.  This equation applies

only  to  thin-walled  closed  tubes  with   constant   wall

thickness.

Page 23 of 58

tm

PC-STRAN    v5.0x  Users Guide

To  compute the approximate torsion constant for thin-walled

closed tubes of any cross-sectional  shape  and  of  varying

wall thickness, t, use the following formula:

2

4 a

TC =  ---------

� (L/t)

i

where a is the whole area bounded by the centerline  of  the

perimeter,  p, of the tube.  The area a is approximately the

average of the two areas enclosed  by  the  inside  and  the

outside  surfaces  of  the tube.  It is the area enclosed by

the centerline of the wall's contour.  The summation �  adds

all  the  ratios of length/thickness.  This equation applies

only to thin-walled closed tubes with varing wall thickness.

To compute the approximate torsion constant for  thin-walled

open  cross sections with varying wall thickness, t, use the

following formula:

1       3

TC =  -  � L t

3  i

where the summation � adds all the products of length  times

thickness  cubed.  This equation applies only to thin-walled

open  cross  sections  with  varing  wall  thickness.   This

equation   applies   to   cross  section  shapes  resembling

C,L,T,I,H,X,Z, etc.

See References section for computing Torsion Constants.

Page 24 of 58

tm

PC-STRAN    v5.0x  Users Guide

PC-MESH

-------

PC-MESH creates a file of the geometry of the structure  to

be analyzed.  The created file is stored on disk and used as

an  input  file to PC-STRAN.  The program allows the user to

view the structure from  any  angle  (see  Some  Definitions

section).   The  program  is divided into two general parts;

one for specifying node (or joint) information and the other

for specifying member information.

PC-MESH can recall a geometric  MESH  file  from  disk,  and

modify  node or member records, add nodes or members, delete

nodes or members, print all node or member records to  disk,

printer  or  screen, and/or file the updated node and member

records.

PC-MESH is menu driven with scroll record data entry.  There

are 10 fields in each node record.  They are:

1) Node Number - 1 to 3 digits

2) Global x coordinate - length

3) Global y coordinate - length

4) Global z coordinate - length

5) Node restraint in global x direction   | 0=restrained

6) Node restraint in global y direction   |  0=default

7) Node restraint in global z direction   | 1=NOT

8) Rotation restraint about global x axis |   restrained

9) Rotation restraint about global y axis |

10) Rotation restraint about global z axis |

There are 6 fields in each member record.  They are:

1) Beginning Node number - 1 to 3 digits

2) Ending Node number - 1 to 3 digits

3) Member connection to beginning node | see

4) Member connection to ending node    | below

5) Member roll angle - degrees

6) Member type name - alphanumeric characters

(NOTE: Can be NODE2NODE, See Section on

Tieing Nodes Together)

Page 25 of 58

tm

PC-STRAN    v5.0x  Users Guide

Member-to-node connection values can have one of ten  single

digit values.

0 is for a swivel connection and is the default value.

1 is for a fixed or rigid connection.

2-9 are for  semirigid  connections  (see  References)  with

fixity  factors  of  .2 to .9 (i.e. the fixity factor is

the  specified  single  digit  divided  by  ten).    The

semirigid connection can be thought of as two rotational

and one torsional springs at the member ends, with:

spring     flexural (or torsional) rigidity       3 FF

constant  = ________________________________   X  ______

element length               1 - FF

Note: The fixity factor (FF) is the same for the two bending

axes AND the torsional axis.

To understand the definition of a roll angle consider that a

member has a LOCAL set of coordinate axes.  The X (caps  for

LOCAL)  axis  runs + from the beginning of the member to its

end passing through the member's centroid.  The  member's  Y

and  Z  axes  define the member's Moments of Inertia Iyy and

Izz. Remember that the LOCAL coordinate  system  has  to  be

right-handed  too.   To  obtain the roll angle translate the

member's beginning node to the Global origin  and  move  the

member's end node to the +x global axis WITHOUT twisting the

member about its' X LOCAL axis.  The angle measured from the

+z  global  axis  to  the +Z LOCAL axis is the member's roll

angle. Special rules  are  required  for  vertical  members.

Foremost  is  that  end  nodes  have  to be higher (in the y

direction) than beginning nodes.  Note  the  roll  angle  is

measured + from +z (toward +x) to +Z.

Page 26 of 58

tm

PC-STRAN    v5.0x  Users Guide

Roll  angle (RA) is the angle the local +Z axis makes with a

line (call it P) which  lies  in  the  cross-section  (i.e.,

local  Y-Z  plane)  and is parallel to the global x-z (i.e.,

horizontal) plane.  The angle is measured positive from line

P toward the local +Z axis.  Mathematically:

^     ^   ^       ^

[ y x ( Y x Z ) ] . Z = cos(RA)

where x is cross product

. is dot product

^ is vector

y

In the example on the right,              |

the vertical member runs                  |       X

o to X. It would have a -10               |       |

degree roll angle or +350                 |       |

degrees.  In the picture                  |       |

z'o is parallel to zO.                    O--Y----|----x

Also Zo is perpendicular                 /    \   |

to Yo.  See also Section                /      \  |

on Computing Roll Angle.               /        \ |

/          \|

With PC-MESH you can view            /           .o

the structure interactively,        z          . /

specifying your line of sight                .  /

(see Some Definitions).  You               .   /

can rotate and also shift your           .    /

line of sight, up, down, left,         Z     z'

right, and zoom in and out of the

structure.  Fixed member connections

are drawn to its nodes while swivel connections  stop  short

of  its  nodes.   Instructions  are  given with the ViewMesh

option in PC-MESH.

See also Section on Command Line Switches/n, and Section  on

Video Override Switch.

PC-MESH has  an  option  to  reduce  or  minimize  the  node

bandwidth.   See  Advanced  Feature.   The node bandwidth is

linearly related to the stiffness matrix semibandwidth.  The

time required to solve a  linear  system  increases  as  the

stiffness  matrix  bandwidth squared (for the same number of

degrees of freedom). Also because the number of  records  is

limited  to  32727,  a  minimum  bandwidth means the fastest

solution and maximum problem size.

Page 27 of 58

tm

PC-STRAN    v5.0x  Users Guide

Computing Roll Angle

--------------------

Because of the possible  difficulty  with  computing  roll

angle,  the following is a BASIC listing of ROLLANGL.BAS for

computing roll angle.  The program is supplied on disk along

with an executable version ROLLANGL.EXE.

10 REM interpretive basic program to calculate rollangle ***

20 DEFINT I : DIM X(8),Y(8),Z(8)

30 PI#=3.141596253#

40 INPUT "    beginning node coordinates: ",X(1),Y(1),Z(1)

50 INPUT "       ending node coordinates: ",X(2),Y(2),Z(2)

60 INPUT "coord's which lie on +XY plane: ",X(3),Y(3),Z(3)

70 X(4)=0 : Y(4)=1 : Z(4)=0       'global vertical or y axis

80 X(6)=0 : Y(6)=0 : Z(6)=1       'global z axis

90 REM *****************************************************

100 I3=5 : I2=2 : I1=1 : GOSUB 260 'shift local X, normalize

110 I3=3 : I2=3 : I1=1 : GOSUB 260 'shift (+XYplane), norm

120 IF X(5)=0 AND Z(5)=0  GOTO 140 'vertical member?

130 I3=6 : I2=4 : I1=5 : GOSUB 290 'horizontal-Z vector,norm

140 I3=7 : I2=3 : I1=5 : GOSUB 290 'local Z vector, norm

150        I2=7 : I1=6 : GOSUB 340 'angl between the two Z's

160 IF (RA=0) OR (RA=180) GOTO 200 'special case Z's paralel

170 I3=8 : I2=7 : I1=6 : GOSUB 290 'is normal = X?

180 I3=8 : I2=8 : I1=5 : GOSUB 260 'subtract to find out

190 IF 1<VL THEN RA=-RA            'if identical=0 else =2

200 PRINT USING "                    roll angle= ####.##";RA

210 GOTO 60                        'try different +XY plane

220 REM ********** normalize vector i3 *********************

230 VL = SQR(X(I3)*X(I3) + Y(I3)*Y(I3) + Z(I3)*Z(I3))

240 IF 0 = VL THEN RETURN

250 X(I3)=X(I3)/VL: Y(I3)=Y(I3)/VL: Z(I3)=Z(I3)/VL : RETURN

260 REM ***** subtract i1 from i2 to get i3, normalize i3 **

270 X(I3)=X(I2)-X(I1): Y(I3)=Y(I2)-Y(I1): Z(I3)=Z(I2)-Z(I1)

280 GOSUB 220 : RETURN

290 REM ***** cross i1 toward i2 to get i3, normalize i3 ***

300 X(I3) =             - Z(I1)*Y(I2) + Y(I1)*Z(I2)

310 Y(I3) = Z(I1)*X(I2)               - X(I1)*Z(I2)

320 Z(I3) =-Y(I1)*X(I2) + X(I1)*Y(I2)

330 GOSUB 220 : RETURN

340 REM ******* get angle between unit vectors : i1,i2 *****

350 DOT = X(I1)*X(I2) + Y(I1)*Y(I2) + Z(I1)*Z(I2)

360 IF DOT = -1 THEN RA = 180 : RETURN

370 IF DOT =  1 THEN RA =   0 : RETURN

380 RA = 90# - ATN(DOT/SQR(1-DOT*DOT)) * 180#/PI# : RETURN

Page 28 of 58

tm

PC-STRAN    v5.0x  Users Guide

Tieing Nodes Together

---------------------

The situation occurs that two continuous members cross  and

where they cross, they have equal displacement but not equal

rotation.  Essentially they are connected with a zero length

pin  with  swivel  connections  at  both  ends.   Think of a

scissors.  Up until Version 5.00, PC-STRAN could not connect

members in such a  fashion.   If  one  connected  everything

rigidly   to   a   node,   the   members  would  have  equal

displacements AND rotations.  Pinning members  to  the  node

would render some members noncontinuous.

With Version 5.00, more than one  node  can  have  the  same

position  in  space.   To  tie  the  nodes together (lateral

displacements not rotations) enter in PC-MESH  edit  Members

the following:

Member   end Fixity  member    Member

Nodes     factor     Roll      Type

beg-end   beg  end   Angle

###-###    #    #    ###.##   ---------

2   5    0    0         0   NODE2NODE

For this example we "tie" node 5 to node  2.   Then  proceed

normally:

NOTE: The USER must follow three rules to tie node B to A.

1) Nodes B and A must have identical coordinates

(i.e., zero length member.)

2) Nodes B and A must have identical lateral node

restraints (rotational restraints can be different.)

3) A- B Member Type must be NODE2NODE.

The nodes will have identical displacements and forces  will

sum  to  the  applied  forces  at the "tied" nodes.  Applied

moments will go the respective nodes.  (See References.)

Page 29 of 58

tm

PC-STRAN    v5.0x  Users Guide

Advanced Feature

----------------

Besides  having  the  option  to  reduce  the  bandwidth  by

resequencing  the  nodes.   PC-MESH  also  has the option of

RELABELING the nodes  and  the  members.   PC-MESH  has  the

option of reordering the members.  The RELABEL option should

be  used  with  EXTREME  CAUTION  since  the node labels and

member labels in the .LDS files are NOT changed.

!!!!! USER BE WARNED! !!!!!

It is suggested that the user apply the  loads  AFTER  using

this RELABEL feature.

Page 30 of 58

tm

PC-STRAN    v5.0x  Users Guide

PC-LOAD

-------

PC-LOAD creates a file  of  applied  loads  to  a  specific

structure. The created file is stored on disk and used as an

input  file  to  PC-STRAN.  This eliminates specifying loads

every time an analysis  is  run.   Eight  different  loading

cases  for  the same structure can be specified in one file.

For each loading case, the members' weight can  be  INCLUDED

or IGNORED.  The members' weight load is calculated by:

member weight = density * cross sectional area * length

acting in the -y GLOBAL direction.

PC-LOAD can recall a LOAD file from disk, and for each  case

modify  load records, add loads, delete loads, print all the

load records to disk, printer or  screen,  and/or  file  the

updated load records.

PC-LOAD is menu driven with scroll record data entry.  There

are 7 fields in each applied node load record.  They are:

1) Node number - 1 to 3 digits

2) Force in global +x direction - force

3) Force in global +y direction - force

4) Force in global +z direction - force

5) Moment about global x axis, + going from y to z -

force*length

6) Moment about global y axis, + going from z to x -

force*length

7) Moment about global z axis, + going from x to y -

force*length

Page 31 of 58

tm

PC-STRAN    v5.0x  Users Guide

There are 5 fields in each applied member load record.  They

are:

1) Beginning Node number - 1 to 3 digits

2) Ending Node number - 1 to 3 digits

3) Value of linearly distributed load at the

beginning node - force/length

4) Value of linearly distributed load at the

ending node - force/length

5) Distributed Load Type - alphabetic characters

Nine  different  choices  of  distributed  load   type   are

permitted:

XA refers to a linearly distributed load in the GLOBAL x

direction per unit of member Actual length.  YA and  ZA  are

similar.

XP refers to a linearly distributed load in the GLOBAL x

direction per unit of member  Projected  length  on  the  yz

plane.  YP and ZP are similar.

XL refers to a linearly distributed load in the LOCAL  X

direction  per  unit of member actual length.  YL and ZL are

similar.

Gravity loads would be YA with negative values.  Snow  loads

would  be  YP  with  negative  values.  Uplift wind pressure

loads might be YL with positive values.

Page 32 of 58

tm

PC-STRAN    v5.0x  Users Guide

PC-STRAN (General) PC-SOLVE (Particular)

----------------------------------------

PC-STRAN analyzes 3 dimensional  structures  whose  members

are essentially one dimensional connected together at joints

or  nodes.  The  connectivity  of  members  to the nodes can

either be frictionless swivels or fixed or  semirigid.   One

end of a member can be fixed and the other swivel.  PC-STRAN

requires as input the following 3 files:

1) a PROPerty file on member types

2) a geometric MESH file with node

and member information

3) a LOAD file specifying loads at the nodes and members

Because each node can have 6 possible degrees of freedom - 3

translation and 3 rotation - care must be taken  in  PC-MESH

to  release  the  nodes.   Consider  the  following types of

structures:

1) Rigid beam   - dx=dz=rx=ry=0  2 DOF's/node (010 001)

2) Plane truss  - dz=rx=ry=rz=0  2 DOF's/node (110 000)

3) Plane frame  - dz=rx=ry=0     3 DOF's/node (110 001)

4) Space truss  - ry=rx=rz=0     3 DOF's/node (111 000)

5) Space frame  -                6 DOF's/node (111 111)

Since  PC-STRAN  can  handle  part  frame  and  part   truss

structures  the  maximum  records  of  the K matrix is not a

multiple of number of nodes but is a separate  maximum  (see

Maximums).   After  PC-STRAN  reads  in  the input files and

checks that the information is consistent (e.g. member types

in MESH file exist in PROPerty file, LOADed nodes  exist  in

MESH  file,  etc.) the total number of records is calculated

and printed.  Output is directed to a  user  specified  file

and includes the following for all load cases:

1) all the input

2) degrees of freedom of the structure

semibandwidth of K matrix

number of records in K matrix

3) equivalent node loads for member loads

4) member end forces

in local coordinate system

5) node movement and forces (reaction & applied)

in global coordinate system

Page 33 of 58

tm

PC-STRAN    v5.0x  Users Guide

As PC-STRAN  is  analyzing,  messages  are  printed  to  the

monitor  (e.g. as member stiffnesses are added to the global

matrix). Member stiffness matrices,  in  local  coordinates,

and  local-to-global coordinate transformation matrices, are

stored in a temporary STORAGE file, so they only have to  be

calculated  once thereby reducing computer time.  The global

matrix file is stored in available EXTENDED or  conventional

DOS memory.

Limitations

-----------

PC-STRAN assumes small  deformations  and  member  direction

cosines  to  remain  the  same  throughout  the  loading and

deformation history. The global matrix (17 digits precision)

is   solved   using   Crout   reduction   and   Gauss-Jordan

elimination,   with   matrix  skyline  profiling.   Buckling

mechanisms, collapse mechanisms, and unstable mechanisms are

NOT taken into account.   PC-STRAN  does  a  linear  elastic

structural analysis.

PC-VIEW

-------

PC-VIEW views  the  deflected  loaded  structure  over  the

original  unloaded structure.  PC-VIEW needs as input a file

with extension DEF created by PC-STRAN during the structural

analysis.  The loaded structure is drawn with dashed  lines.

Two  additional  features  are  incorporated  in PC-VIEW not

found in PC-MESH.  They are a SCALE adjustment to  make  the

node  displacements  bigger  or  smaller  and  the LOAD CASE

specification (0 to 8 max).   Only  node  displacements  are

graphed  not member bending. Instructions are given with the

ViewMesh option in PC-VIEW.

PC-VIEW has option f) to define the load factors multiplying

the results for load cases 1-8.   The  combination  is  load

case 0.

PC-VIEW also has a graphics option with instructions to view

load, shear, moment, slope and displacement diagrams of  the

structure's members.

See also Section on Command Line Switches/n, and Section  on

Video Override Switch.

Page 34 of 58

tm

PC-STRAN    v5.0x  Users Guide

PC-RPT

--------

PC-RPT writes the structural analysis report.  PC-RPT needs

as input a file  with  extension  RES  created  by  PC-STRAN

during  the structural analysis.  PC-RPT creates a file with

extension RPT which is an ASCII file.  This file can be sent

to a printer  with  the  DOS  print  command  (e.g.  A>print

filename.RPT)  or  to screen with the DOS more command (e.g.

A>more<filename.RPT). PC-RPT is  provided  in  BASIC  source

code  so  you  can "customize" your reports.  To run  PC-RPT

you have to be in  BASIC (e.g A>basica ).   When  in  BASIC,

LOAD"pc-rpt"  and  press  the RUN key.  Before you customize

PC-RPT it is suggested you  make   a   copy   of   the   one

provided  and  customize the copy. REMEMBER you have to read

everything from filename.RES in the order  given  in  PC-RPT

but you can write to filename.RPT in any fashion you wish.

Member forces are in relation to  LOCAL  member  coordinate

systems.   Internal  minimum  and  maximum  forces along the

length of a member are only printed if there is  a  linearly

distributed  load  (otherwise  they occur at the ends of the

member.)

Node forces and movement are  in  relation  to  the  GLOBAL

coordinate system.

Load Factor Analysis

--------------------

PC-RPT can also provide limitless linear combinations of the

eight possible load cases.  This is quite  useful  for  Load

and  Resistance  Factor Design (LRFD).  After the individual

load cases are printed  to  file,  PC-RPT  will  prompt  for

factors  to  multiply  each  load  case and print the linear

additions of the results into  the  *.RPT  file.   The  user

might designate load case 1 for permanent loads, load case 2

for floor live loads, load case 3 for snow loads, etc.  This

then  allows  the  user to combine the load cases in any way

desired.   Entering  ALL  zeroes  for   the   load   factors

terminates PC-RPT.

[Note: To get just the weight of the  structure  as  a  load

case,  put  on  a  zero  force concentrated load and specify

weight included option!]

PC-VIEW has option f) to define the  load  factors  for  the

different load cases 1-8.  The load combination becomes load

case 0.

Page 35 of 58

tm

PC-STRAN    v5.0x  Users Guide

Graphics Hardcopy

-----------------

Using DOS 2.x or later and an IBM dot  matrix  printer  (or

one that will understand the graphics commands) you can do a

graphics screen print. Before running PC-STRAN type GRAPHICS

while  in  DOS command (e.g. A>graphics ). Then while in the

graphics of PC-MESH or PC-VIEW you can print the  screen  by

pressing  the  Shift-PrtSc  keys.   Refer to your DOS manual

GRAPHICS Command.

HIRESPIX is provided and is similar to  IBM's  GRAPHICS  but

prints  a  smaller  graph.   Install HIRESPIX instead of, or

AFTER, GRAPHICS.

HIRESPIX works with CGA, EGA, VGA, MCGA, HGC and HGC+ and an

IBM dot matrix printer or compatible.

Run-Time Errors

---------------

To reduce the size of the .EXE modules QuickBASIC's ON ERROR

GOTO was eliminated.  Check to make  sure  drive  doors  are

closed,  printer  is  turned on,  etc.  The modules do check

for input files existing, as well as,  87  math  coprocessor

exception    conditions   (invalid   operation,   underflow,

overflow, divide by zero, etc.)

If you get a  divide  by  zero,  overflow  or  any  87  math

coprocessor  error  during  "Crout  reduction & Gauss-Jordan

elimination" then  your  stiffness  matrix  is  most  likely

singular.  Check your fixity factors and node restraints.  A

member  has  to  be restrained from spinning about its own X

axis!  A member that intersects another member but is  given

a  fixity factor of zero does NOT prevent either member from

spinning about  their  own  X  axes!   Watch  out  also  for

collapse  mechanisms  in  trusses  as  well  as any buckling

mechanisms!

If you get the error - Bad file number at 710 - when using

PC-RPT.BAS,  load  BASICA  with  the  switch   /f:4,   i.e.,

A>BASICA/f:4 .

Page 36 of 58

tm

PC-STRAN    v5.0x  Users Guide

References

----------

Przemieniecki, J.S.

1968.  Theory of Matrix Structural Analysis.

McGraw-Hill Book Co., Inc.,

New York. (see pp.61-82)

Timoshenko, S. and J.N. Goodier

1951.  Theory of Elasticity.

Second Edition, McGraw-Hill Book Co., Inc.,

New York. (see p.267)

Seely, F.B. and J.O. Smith

1955. Advanced Mechanics of Materials.

Second Edition, John Wiley & Sons, Inc.,

New York. (see pp.276-279)

Popov, E.P.

1968.  Introduction to Mechanics of Solids.

Prentice-Hall, Inc.,

Englewood Cliffs.

Isaacson, E. and H.B. Keller

1966.  Analysis of Numerical Methods.

John Wiley & Sons, Inc.,

New York.  (see pp.50-52)

Willems, N. and W.M. Lucas, Jr.

1978.  Structural Analysis for Engineers.

McGraw-Hill Book Co., Inc.,

New York. (see Appendix D - torsional constants)

Page 37 of 58

tm

PC-STRAN    v5.0x  Users Guide

Grooms, H.R.

Algorithm for Matrix Bandwidth Reduction.

1972.  Journal of the Structural Division,

Proceedings of the American Society of Civil Engineers,

Vol. 98, No. ST1, pp.203-214.

Wang, C.K.

1983.  Intermediate Structural Analysis.

McGraw-Hill Book Co., Inc.,

New York. (see  Chapter  20,  Rigid  Frames  with  Semirigid

Connections)

Cook, R.D.

1974.  Concepts and Applications of Finite Element Analysis.

John Wiley and Sons, Inc.,

New  York.   (see  Section  11.8  -  Enforcing  Equality  of

Displacements)

Cook, R.D. and W.C. Young

1985.  Advanced Mechanics of Materials.

Macmillan Publishing Company

New York.  (see pp.292-296 - torsion formulas)

Page 38 of 58

tm

PC-STRAN    v5.0x  Users Guide

PC-STRAN Versions / Enhancements

--------------------------------

1.0  -  First documented stand alone programs

-  PC-PROP, PC-MESH, PC-LOAD, PC-3DSA

-  64 member types,64 nodes,64 members,80 dof's

1.2  -  PC-3DSA renamed to PC-STRAN

-  programs chained

-  128 nodes,256 members,254 dof's

-  disk storage of WORK files

1.4  -  upgrade of 1.2

-  192K of memory

2.0  -  add PC-VIEW, PC-WRITE

-  255 dof's,8 load cases

-  transfer information between programs

2.5  -  upgrade of graphics

-  linearly distributed member loads (128)

-  members' weight load option (for all members)

-  moment/shear diagrams for members (256)

-  scroll method of selection

-  greater than 255 dof's

2.57 -  8087 version of 2.5

3.00 -  add PC-SOLVE as subprogram to PC-STRAN

-  direct memory allocation algorithm for K matrix &

load vectors (resulting in significant speed

improvement)

-  use of 16 bytes 0:04F0-0:04FF for data passing

between program modules

-  autocheck and autoutilization of 87 math

coprocessing chip

3.10 -  show nodes option in PC-MESH and PC-VIEW

-  finer Zoom increments in PC-MESH and PC-VIEW

-  course and fine Shift adjustments in PC-MESH

and PC-VIEW

-  assembly language profile routine in PC-SOLVE

3.20 -  PC-WRITE renamed to PC-RPT

-  node bandwidth minimization (reduction) option

in PC-MESH

-  capabilities for semirigid joints

3.22 -  command line switches for non-8087 and

non-EGA compatibles

3.50 -  expanded drive letters A-Z

-  compiled with QuickBASIC 4.0

(runs faster though larger programs)

-  infinite number of linear combinations

of load cases

in PC-RPT (useful in load factor analysis)

-  PC-STRAN Main Menu in each program module

Page 39 of 58

tm

PC-STRAN    v5.0x  Users Guide

4.00 -  path capabilities for DATA, PROGRAM, STORAGE

and WORK files

-  PC-STRAN is a parent process with children,

PC-PROP, PC-MESH, PC-LOAD, PC-SOLVE and PC-VIEW

-  PC-RPT incorporates user-defined header

and page numbering

-  data entry uses scrolling of records

in editing window,  data is not reformatted

-  graphics and HiResPix (screen dump driver)

works with CGA, EGA, VGA, HGC and HGC+

and 100% compatibles

4.01 -  fixes bug in the creation of .PRP, .MSH

and .LDS files

4.02 -  fixes bug (only on 8088 machines)

in editing features

-  checks for errors before writing .PRP, .MSH

and .LDS files

zeroes blank fields before writing files

-  fixes bug in diagrams in PC-VIEW for HGC

and HGC+ graphics

4.10 -  fixes bugs in PC-LOAD and PC-MESH

-  load combination in PC-VIEW as Load Case #0

-  RELABEL option in PC-MESH

-  more consistent Editing and Scrolling Keys

-  major revision of Documentation with INDEX

-  separate INVOICE.DOC and PROBLEM.DOC

-  CFGSTRAN to configure default \Paths\

in PC-STRAN.EXE

-  EFILECHK for .EXE file integrity check

-  VIDFIX.EXE for early-IBM BIOS video bug

-  works with MCGA graphics

-  horizontal-to-vertical scaling override switch

-  checks 87 math coprocessor for exception

conditions (i.e. errors)

4.30 -  (Microsoft) mouse support

-  min and max forces are calculated in PC-RPT

-  PC-RPT.EXE and PC-RPT.BAS are provided

-  start new problem w/o leaving PC-STRAN

-  large (MASTER).PRP file w/ max 512 entries

-  higher resolution in MCGA graphics

-  INSTALL.BAT for hard disk users

-  compressed EXE files

-  \DATA\ subdirectory listing

-  PC-PROP, PC-MESH and PC-LOAD will not prompt

"Do you want to return ...?" if you file the

records after editing them

-  beeps in editing window changed to low beeps

and eliminated for field widths of 1

Page 40 of 58

tm

PC-STRAN    v5.0x  Users Guide

4.31 -  correct density in distributed (MASTER).PRP

-  correct PC-RPT.BAS and PC-RPT.EXE

for load factor analysis

5.00 -  add ability to use EXTENDED memory thereby

relaxing Number of Records limitation

-  add ability to TIE nodes together

(i.e., zero length truss member)

-  ROLLANGL.BAS and ROLLANGL.EXE programs

to calculate member rollangle.

Obtaining the Latest Version

----------------------------

The latest version of PC-STRAN will be posted  on  CIME-ISE

in  the file STRAN50#.ZIP.  The # is a minor revision number

0 through 9.   I  will  notify  registered  users  of  major

revisions.

CIME-ISE  is  the electronic bulletin board system (BBS) of

the American Society of Mechanical Engineers.   The  acronym

stands for Computers In Mechanical Engineering - Information

and   Software  Exchange.   CIME-ISE  is  on-line  24  hours

everyday.  The system answers 5 phone lines  at  (608)  233-

3378  using  300,  1200,  2400,  or 9600 baud.  The CIME-ISE

library currently has more than 4500 programs of interest to

Mechanical Engineers.  All software can  be  freely  copied.

There  are no usage or registration fees involved and anyone

interested in technical or scientific software is invited to

call.

Page 41 of 58

tm

PC-STRAN    v5.0x  Users Guide

Some Questions Answered

-----------------------

1.  "Will it help if I buy more  RAM  memory  for  my  PC?".

ANSWER:  Yes,  if  with  enough  memory  and purchase of the

appropriate software you can  create  a  "RAM"  (electronic)

disk  drive  to hold the TEMPorary storage WORK files needed

by PC-STRAN (See Section on Use of EXTENDED Memory).

2.  "In what language was PC-STRAN  written?".   ANSWER:  It

was  written  in  Microsoft  QuickBASIC,  then compiled with

Microsoft's QuickBASIC 4.5 Compiler and linked with Cresent

Software's QuickPak Professional Library.

3.  "Can  I  obtain  the  source  code  for  the  program?".

ANSWER: No.

4.  "What is your update policy?".  ANSWER: Presently I will

notify registered users of updates by postcard.  Or you  can

mail  me  two  diskettes with postage and mailer, or a check

for $20, every six to nine months or so, and I will send you

the then-current version.

5.   "Do  you  have  any  other  programs   that   you   are

distributing?".  ANSWER:  Yes,  PCBRIDGE(tm) is a structural

analysis program for a continuous beam with  moving  vehicle

load envelopes.

6.  "Is there a version of PC-STRAN  for  other  computers?"

ANSWER: No.

7.  "What about  program  support?".  ANSWER:  I  will  give

support  and  reply  to  written  questions  from registered

users.  I will also notify registered users of  any  program

problems.

Page 42 of 58

tm

PC-STRAN    v5.0x  Users Guide

Permission to COPY

------------------

Clubs, other non-profit organizations and  vendors  approved

by  the  Association  of  Shareware  Professionals (ASP) are

granted permission  by  the  author  to  freely  copy  these

programs  and  documentation and share it with their members

and customers, so long as:

1.  No price is charged for the software  or  documentation.

However,  a distribution cost may be charged for the cost of

the diskettes, so long  as  it  is  not  more  than  $5  per

diskette.

2.  Club  members  and  ASP-approved  vendor  customers  are

informed  of  the  user-supported  concept and encouraged to

support it with their registrations.

3.  The program or documentation are not modified in any way

and are distributed together.

[NOTE: If you receive the software  through  a  club,  other

non-profit  organization,  friend or an ASP-approved vendor,

you have NOT registered the software with the  author!   You

MUST  register  the  software with the author by sending the

registration fee to the author!]

See User-Supported Software.

Page 43 of 58

tm

PC-STRAN    v5.0x  Users Guide

User-Supported Software

-----------------------

User-Supported Software

If you are using this program

your registration, $55, is required.

Joe Murphy

Post Office Box 56164

Madison,  WI  53705

You are encouraged to copy and share these programs.

tm

PC-STRAN   is NOT public domain or free  software,   but  is

being distributed as "user-supported" software.

Non-registered users of this software are granted a  limited

license  to  make  an  evaluation  copy  for  trial use on a

private, non-commercial basis,  for the express  purpose  of

determining  whether  the  programs  are  suitable for their

needs.  At the end of this trial period, you  should  either

register your copy or discontinue using the programs.

This means that if you use this program for any real benefit

to yourself or others, you should pay for your  copy.   That

way  the author will be able to provide you with support and

updates.

A registration entitles you to use the program  on  any  and

all   computers   available  to  you,   with  the  following

restriction:  If others have access to this program  or  may

use it,  then you should purchase a site license.

Page 44 of 58

tm

PC-STRAN    v5.0x  Users Guide

All users are granted a limited license to copy the programs

for  the  trial  use  of others and subject to the following

limitations.  This license does NOT include distribution  or

copying of this software package:

1. In connection with any other product or service.

2. For general use within a company or institution.

3. For any consideration fee.

4. Distribution in modified form,

i.e., the documentation must be included.

User-supported software is a way  in  distributing  computer

programs, based on these beliefs:

1.  That the value and utility of software is best  assessed

by the user on his/her own system.

2.  That the creation of personal computer software can  and

should be supported by the computing community.

3.  That distribution  of  programs  should  be  encouraged,

rather than restricted.

Anyone may request a copy of  a  user-supported  program  by

sending  blank,  formatted  disk(s)  to  the  author  of the

program.  An  addressed,  postage-paid  return  mailer  must

accompany  the  disk  (no exceptions, please). A copy of the

program, with documentation on the disk,  will  be  sent  by

return  mail.   The  program  will  carry  a notice with the

registration fee and the program's  author.   A  person  who

uses the program is expected to become a registered user.

Free distribution of software and registration for  its  use

eliminates   costs   for  advertising  and  copy  protection

schemes.

Users obtain quality software at a  reasonable  price.  They

can  try  it  out before buying, and do so at their own pace

and in the comfort of their own home or office.

User-supported  software  only  works  if  the  software  is

actually  user  supported.   If  you  use  the software, you

should show your support and register your copy.  It's  only

through  your  registration  that  the  author can afford to

improve the software and bring you  improved  versions.   If

you  don't  like  the  software,   or feel that it is sorely

lacking in some area(s),  then please don't hesitate to  let

the author know.

Page 45 of 58

tm

PC-STRAN    v5.0x  Users Guide

Shareware

---------

Shareware is a unique method of marketing  software  based

on the try-before-you-buy concept.  The authors of shareware

programs  retain all rights to the software (copyright), and

they require a "license fee" from those persons who continue

to use the software.  The term shareware goes  back  to  the

early  1980's  when  there was a column in Infoworld by that

name  describing  programs  available   through   electronic

Bulletin  Board Systems.  In 1984, Nelson Ford had a contest

in his Softalk column on user-supported  and  public  domain

software.   The purpose of this contest was to find a better

name than "user-supported" for  this  software  genre.   The

overwhelming   favorite   was  shareware  and  he  announced

shareware the winner.  Since then, the name has increased in

use by the common public.  Shareware is a concept  that  has

wide  respect  among  part  of  the user community who never

doubted that it is a generic term.

The   Association   of   Shareware    Professionals    was

incorporated  in  August,  1987, to strengthen the future of

shareware software as an alternative to  marketed  software.

This   organization   consists   of  shareware  authors  and

associate  vendor  members  nationwide  who  desire  to  see

recognized standards for quality software and documentation.

For  more  information,  write to:  Association of Shareware

Professionals,  545 Grover Road, Muskegon, MI 49442.

_______

____|__     |               (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

Joe Murphy is a member of  the  Association  of  Shareware

Professionals  (ASP).   ASP  wants  to  make  sure  that the

shareware principle works for you.  If  you  are  unable  to

resolve  a  shareware  related problem with an ASP member by

contacting the member directly,  ASP may be  able  to  help.

The  ASP Ombudsman can help you resolve a dispute or problem

with an ASP member, but does not provide  technical  support

for members' products.  Please write to the ASP Ombudsman at

545  Grover  Road,  Muskegon,  MI 49442 or send a Compuserve

message via easyplex to ASP Ombudsman 70007,3536 .

Page 46 of 58

tm

PC-STRAN    v5.0x  Users Guide

Documenting Problems

--------------------

If in using this shareware software you encounter a problem,

(software   bugs,   documentation   errors,   or   suggested

enhancements)  and  after  rereading  the  documentation and

after calling the author for help,   send  DOCumentation  of

the  problem  with input files (on disk) to the author along

with the filled out  form  in  PROBLEM.DOC.   This  way  the

author can try to resolve the problem.

Invoice

-------

If you have need for an invoice so  that  you  can  initiate

payment  through  your company accounting system, an invoice

is provided in  INVOICE.DOC.   For  those  of  you  in  this

category,  or  for  those  who  wish an invoice for your tax

records, it should serve the purpose.

Page 47 of 58

tm

PC-STRAN    v5.0x  Users Guide

8087 Version

------------

PC-STRAN v3.00 and later checks if a  87  math  coprocessing

chip  is  available.   If  PC-STRAN  detects it is, PC-STRAN

branches to code that utilize assembly language  subroutines

written  for  the  87 chip during the number crunching Crout

reduction & Gauss-Jordan elimination in PC-SOLVE. See below.

Command Line Switches/n

-----------------------

tm

PC-STRAN   has an override switch at the command  line.   To

use the override option, type at the command line

pc-stran/n

n     [100% IBM CGA] [87 in PC clone] [transfer prompts]

1          no

2                         no

3          no             no

4                                          no

5          no                              no

6                         no               no

7          no             no               no

[100% IBM CGA]

On some non-IBM CGA and EGA cards and MCGA the node  number

option  in  PC-MESH  and PC-VIEW produces numbers with lines

through the numbers. If this is the case use option 1,3,5 or

7.

[87 in PC clone]

On some PC clones the program senses  an  87  chip  present

when  one  is not installed.  If this is the case use option

2,3,6 or 7.

[transfer prompts]

When returning to PC-STRAN main menu  the  program  default

prompts  "Do  you  want  to return ...? Y/N".  To turn these

default prompts off use option 4,5,6 or 7.

Page 48 of 58

tm

PC-STRAN    v5.0x  Users Guide

Video Override Switch

---------------------

PC-MESH and PC-VIEW automatically  sense  the  monitor  and

graphics  card  adapter  and assign a horizontal-to-vertical

scaling factor so that a square box will be close  to  being

square.   The default horizontal-to-vertical scaling factors

are:

2.40 for CGA and MCGA adapters and monitors,

1.37 for EGA and VGA adapters and monitors,

1.43 for Hercules adapter and monochrome monitor.

If you want to fine tune or override the default horizontal-

to-vertical scaling factor,  type at the command line

pc-stran  (h2v)

where h2v is the horizontal-to-vertical  scaling  factor  (a

floating point number).

[NOTE: The two override switches can  be  used  together  or

separately, e.g., pc-stran/4 (1.20), pc-stran/4,

pc-stran (1.20), etc.]

Mouse Switch

------------

To use PC-STRAN mouse (Microsoft-compatible)  capabilities,

invoke  the program either with an "m" or "M" on the command

line  (e.g.,  C>pc-stran/m)  if  the  mouse  driver  is  not

installed PC-STRAN will beep but will continue.

Page 49 of 58

tm

PC-STRAN    v5.0x  Users Guide

Test Machines

-------------

PC-STRAN version 5.00 has been developed and tested  on  the

following machines:

IBM PC1 64K motherboard, upgraded BIOS

CGA monitor

V20 chip, 4.77MHz 8087

PC DOS 3.00

ALR 386/220

VGA monitor, Hercules Graphics Card Plus

and monochrome monitor

80386 and 80387 20MHz

PC DOS 5.00

DATAVUE Snap 1+1

V20 and 8087 4.77/9.54MHz

MS DOS 3.30

IBM PS/2 Model 25 (MCGA graphics)

8086 and 8087 8MHz

PC DOS 4.00

Page 50 of 58

tm

PC-STRAN    v5.0x  Users Guide

Index

-----

adapter, 49

ADDED.DOC, 17

algorithm, 38, 39

allocation, 39

alphabetic, 18, 32

alphanumeric, 22, 25

answer, 2, 7, 10, 41, 42

approximation, 23

ASCII, 10, 12, 17, 35

ASP, 43, 46

assembly, 39, 48

autocheck, 39

automatically, 49

autoutilization, 39

axes, 26, 36

axial, 5

axis, 6, 22, 25, 26, 27, 28, 31, 36

bandwidth, 12, 15, 27, 30, 33, 38, 39

BASICA, 10, 35, 36

BBS, 41

beam, 4, 33, 42

bending, 4, 5, 22, 26, 34

BIOS, 20, 40, 50

boot, 9, 16

buckling, 34, 36

bug, 40, 47

capabilities, 39, 40, 49

cartesian, 6

catalog, 22

centroid, 26

CFGSTRAN.EXE, 11, 18

CGA, 5, 18, 36, 40, 48, 49, 50

check, 11, 12, 15, 17, 18, 33, 36, 39, 40, 42, 48

CIME-ISE, 41

clone, 48

collapse, 34, 36

column, 5, 19, 21, 46

compatibility, 2, 20, 21

compatible, 18, 21, 36, 39, 40, 49

compuserve, 46

concentrated, 5, 35

concept, 38, 43, 46

conditions, 36, 40

CONFIG.SYS, 16

configure, 17, 18, 40

Page 51 of 58

tm

PC-STRAN    v5.0x  Users Guide

connection, 5, 12, 25, 26, 27, 29, 38, 45

connectivity, 12, 33

consistent, 11, 33, 40

constant, 2, 4, 22, 23, 24, 26, 37

continuous, 4, 29, 42

coordinate, 4, 6, 25, 26, 28, 29, 33, 34, 35

coprocessor, 36, 40

copyright, 1, 16, 46

corrupt, 2, 11, 18

cross-section, 23, 24, 27

Crout, 34, 36, 48

.DEF, 10, 18

default, 6, 7, 11, 12, 18, 25, 26, 40, 48, 49

deflected, 10, 18, 34

deformation, 9, 34

degree, 15, 25, 27, 33

density, 4, 22, 31, 41

design, 11, 35

details, 2, 5, 22

device=HIMEM.SYS, 16

device=XMSDISK.SYS, 16

diagrams, 19, 34, 39, 40

dimensional, 1, 4, 6, 19, 33

disclaimer, 2, 3

displacement, 4, 5, 14, 19, 29, 34, 38

dispute, 46

DOC.BAT, 17

documentation, 11, 17, 40, 43, 45, 46, 47

dof, 15, 33, 39

DOS, 5, 7, 10, 14, 15, 19, 34, 35, 36, 50

dump, 40

easyplex, 46

editing, 10, 40

EFILECHK.EXE, 18

EGA, 5, 18, 32, 36, 39, 40, 48, 49

elastic, 34, 37

elasticity, 37

electronic, 16, 41, 42, 46

element, 26, 38

elimination, 34, 36, 48

engineer, 3, 5, 37, 38, 41

enhancement, 2, 17, 39, 47

equivalent, 33

erase-me.mb, 15

erase-me.nd, 15, 20

erase-me.rot, 15, 20

erase-me.tmp, 15, 20

error, 2, 12, 17, 23, 36, 40, 47

Page 52 of 58

tm

PC-STRAN    v5.0x  Users Guide

example.LDS, 18

example.MSH, 18

example.PRP, 18

exception, 36, 40, 45

EXTENDED, 2, 7, 11, 15, 16, 20, 34, 41, 42

fields, 22, 23, 25, 31, 32, 40

filename.DEF, 18

filename.LDS, 18

filename.MSH, 18, 19

filename.PRP, 18, 22

filename.RES, 18, 19, 20, 35

filename.RPT, 18, 19, 35

fixed, 26, 27, 33

fixity, 4, 12, 26, 29, 36

flexural, 26

force, 4, 5, 11, 22, 29, 31, 32, 33, 35, 40

force*length, 31

force/length, 22, 32

formula, 23, 24

frame, 4, 5, 33, 38

free, 5, 15, 27, 33, 41, 43, 44, 45

frictionless, 5, 33

function, 17

Gauss-Jordan, 34, 36, 48

geometry, 12, 18, 19, 25

global, 6, 20, 25, 26, 27, 28, 31, 32, 33, 34, 35

graphics, 2, 14, 18, 19, 34, 36, 39, 40, 49, 50

GRAPHICS.COM, 18

graphs, 19

gravity, 32

hardcopy, 2, 36

Hercules, 1, 2, 14, 19, 49, 50

HGC, 5, 18, 19, 36, 40

HIMEM.SYS, 16, 20

HIRESPIX.EXE, 18

history, 34

horizontal, 27, 28, 40, 49

horizontal-to-vertical, 40, 49

IFDIR.COM, 17

inertia, 4, 22, 26

install, 2, 5, 7, 9, 10, 14, 16, 17, 36, 40, 48, 49

INSTALL.BAT, 7, 40

INSTALL?.BAT, 17

instructions, 10, 27, 34

interactive, 20, 27

interior, 5

Page 53 of 58

tm

PC-STRAN    v5.0x  Users Guide

intersects, 36

INVOICE.DOC, 17, 40, 47

Iyy, 23, 26

Izz, 23, 26

joint, 4, 5, 25, 33, 39

knowledge, 2, 3, 5

lateral, 4, 29

latitude, 6, 14

.LDS, 7, 10, 11, 12, 18, 20, 21, 30, 40

length/thickness, 24

license, 44, 45, 46

limitation, 2, 3, 34, 41, 45

linear, 4, 5, 15, 27, 32, 34, 35, 39

live, 35

load, 2, 4, 5, 9, 10, 11, 12, 13, 15, 16, 18, 19, 20, 21,

30, 31, 32, 33, 34, 35, 36, 39, 40, 41, 42

local, 20, 22, 26, 27, 28, 32, 33, 34, 35

local-to-global, 20, 34

longitude, 6, 14

LRFD, 35

mail, 42, 45

manual, 36

(MASTER).PRP, 15, 18, 22, 40, 41

material, 4, 37

maximum, 2, 15, 18, 27, 33, 35

MCGA, 5, 18, 36, 40, 48, 49, 50

member, 4, 5, 9, 12, 15, 18, 19, 20, 22, 25, 26, 27, 28,

29, 30, 31, 32, 33, 34, 35, 36, 39, 41, 43, 46

member-joint, 4

member-to-node, 26

mesh, 2, 4, 6, 9, 10, 11, 12, 13, 14, 18, 19, 22, 25, 27,

29, 30, 33, 34, 36, 39, 40, 48, 49

method, 4, 37, 39, 46

minimization, 39

minimum, 12, 27, 35

module, 36, 39

modulus, 4, 22

moment, 4, 5, 19, 22, 26, 29, 31, 34, 39

moment/shear, 39

monitor, 5, 34, 49, 50

monochrome, 49, 50

monographics, 2, 14, 19

mouse, 2, 3, 13, 14, 40, 49

movement, 12, 13, 14, 33, 35

.MSH, 7, 10, 11, 12, 18, 19, 40

MSHERC.COM, 19

Page 54 of 58

tm

PC-STRAN    v5.0x  Users Guide

node, 2, 4, 5, 6, 9, 12, 15, 18, 19, 22, 25, 26, 27, 28,

29, 30, 31, 32, 33, 34, 35, 36, 39, 41, 48

NODE2NODE, 22, 25, 29

non-EGA, 39

non-IBM, 48

non-loaded, 18

non-normal, 2, 15

noncontinuous, 29

normal, 2, 15, 28, 29

ombudsman, 46

option, 9, 10, 11, 12, 13, 15, 17, 18, 27, 30, 34, 35, 39,

40, 48

origin, 6, 19, 26, 34

overflow, 36

override, 3, 27, 34, 40, 48, 49

paginate, 19

parallel, 27

parent, 13, 19, 40

particular, 2, 33

path, 2, 5, 7, 9, 10, 11, 14, 17, 18, 40

PATHS.SCR, 17

payment, 47

PC-DIR6.EXE, 19

PC-LOAD.EXE, 19

PC-MESH.EXE, 19

PC-PAGE.EXE, 10, 19

PC-PROP.EXE, 19

PC-RPT.BAS, 19, 36, 40, 41

PC-RPT.EXE, 19, 40, 41

PC-SOLVE.EXE, 19

PC-STRAN.DOC, 17

PC-STRAN.EXE, 7, 11, 18, 19, 40

PC-VIEW.EXE, 19

PCBRIDGE(tm), 42

permission, 3, 43

perpendicular, 27

phone, 17, 41

plane, 6, 27, 28, 32, 33

policy, 42

positive, 6, 27, 32

precision, 34

pressure, 32

principle, 46

print, 10, 17, 18, 22, 25, 28, 31, 33, 34, 35, 36

PROBLEM.DOC, 17, 40, 47

procedure, 4

Page 55 of 58

tm

PC-STRAN    v5.0x  Users Guide

processor, 12, 36, 40

profiling, 34

projected, 32

projection, 6

prompt, 7, 9, 10, 14, 35, 40, 48

properties, 4, 19, 22

property, 4, 18, 22, 33

protection, 45

.PRP, 7, 10, 11, 12, 15, 18, 22, 40, 41

purpose, 3, 44, 46, 47

quality, 38, 45, 46

question, 2, 7, 11, 42

QUICKBASIC, 36, 39, 42

ram, 1, 2, 3, 4, 5, 7, 8, 9, 11, 14, 15, 16, 17, 18, 19,

20, 21, 22, 25, 28, 33, 34, 38, 39, 40, 41, 42, 43,

44, 45, 46, 48, 49

ramdisk, 16, 20

reaction, 33

README1.DOC, 17

README1.EXE, 17

record, 10, 11, 15, 22, 25, 27, 31, 32, 33, 40, 41, 47

reduction, 34, 36, 38, 39, 48

reentrant, 23

references, 2, 4, 24, 26, 29, 37

reformatted, 20, 40

register, 1, 41, 42, 43, 44, 45

registration, 17, 41, 43, 44, 45

relabel, 30, 40

release, 14, 33

rename, 21, 39

reordering, 30

repeated, 12

reply, 42

report, 10, 18, 19, 35

representation, 6

require, 2, 5, 6, 22, 26, 27, 33, 44, 46

.RES, 10, 18, 19, 20, 35

resequence, 12

resequencing, 30

resistance, 35

restrained, 25, 36

restraint, 4, 12, 25, 29, 36

restriction, 44

result, 9, 18, 20, 34, 35, 39

revision, 40, 41

rewrite, 21

right-handed, 26

rights, 46

Page 56 of 58

tm

PC-STRAN    v5.0x  Users Guide

rigid, 5, 26, 29, 33, 38, 39

rigidity, 26

roll, 2, 4, 13, 20, 22, 25, 26, 27, 28, 29, 31, 39, 40

ROLLANGL.BAS, 20, 28, 41

ROLLANGL.EXE, 20, 28, 41

rotate, 6, 27

rotating, 12, 19

rotation, 4, 20, 22, 25, 26, 29, 33

.RPT, 10, 18, 19, 35

rules, 26, 29

run-time, 2, 36

scale, 34

scaling, 40, 49

scissors, 29

scroll, 2, 13, 22, 25, 31, 39, 40

section, 4, 5, 9, 15, 18, 19, 20, 22, 23, 24, 25, 27, 31,

34, 38, 42

semibandwidth, 15, 27, 33

semirigid, 26, 33, 38, 39

shapes, 24

Shareware, 3, 43, 46, 47

shear, 4, 19, 22, 34, 39

singular, 36

site, 44

skyline, 34

slope, 19, 34

snow, 32, 35

solution, 4, 27, 40

solve, 2, 9, 10, 17, 18, 19, 27, 33, 34, 39, 40, 46, 47, 48

source, 19, 20, 35, 42

specific, 4, 31, 34

speed, 39

spinning, 36

spring, 26

stand-alone, 17, 18, 19, 20

standards, 46

stiffness, 4, 20, 27, 34, 36

storage, 2, 9, 14, 15, 20, 34, 39, 40, 42

STRAN50#.ZIP, 41

structural, 1, 4, 5, 19, 34, 35, 37, 38, 42

structure, 4, 5, 6, 10, 12, 18, 19, 25, 27, 31, 33, 34, 35

subdirectory, 5, 7, 11, 19, 40

suggestions, 17

support, 3, 40, 42, 43, 44, 45, 46

switches, 3, 19, 27, 34, 39, 48, 49

swivel, 5, 26, 27, 29, 33

system, 2, 5, 6, 7, 9, 26, 27, 33, 35, 41, 45, 46, 47

Page 57 of 58

tm

PC-STRAN    v5.0x  Users Guide

technical, 41, 46

techniques, 2, 11, 13

technology, 1

terminate-and-stay-resident, 19, 20

termination, 2, 15, 20

theory, 3, 37

thin-walled, 23, 24

tips, 2, 11

torsion, 2, 4, 22, 23, 24, 26, 37

transformation, 34

translate, 26

translation, 33

truss, 4, 5, 9, 33, 36, 41

truss/frame, 4, 5

TSR, 18, 19, 20

twisting, 5, 22, 26

underflow, 36

unit, 11, 28, 32, 45, 46

units, 11

unloaded, 19, 34

unstable, 34

update, 10, 12, 19, 20, 21, 22, 25, 31, 42, 44

upgrade, 39, 50

uplift, 32

user-defined, 40

user-supported, 3, 43, 44, 45, 46

vector, 27, 28, 39

vectors, 28, 39

vehicle, 42

vertical, 6, 26, 27, 28, 40, 49

VGA, 5, 18, 36, 40, 49, 50

video, 3, 27, 34, 40, 49

VIDFIX.EXE, 20, 40

viewmesh, 12, 27, 34

viruses, 2, 11, 18

warranty, 3

weight, 31, 35, 39

wind, 32, 40

world, 6, 46

write, 21, 35, 39, 46

XMM, 16

XMS-aware, 16

XMSDISK.SYS, 16, 20

zero, 16, 29, 35, 36, 40, 41

zoom, 12, 14, 19, 27, 39

Page 58 of 58