________tm

PCBRIDGE    v2.8x  Users Guide

________tm

PCBRIDGE

Moving Loads Continuous Beam

BRIDGE analysis program

For the IBM Personal Computer

(C)Copyright 1986,87,88,89,90,96

by Joe Murphy,Ph.D.,P.E.

Version 2.80

Joe Murphy

P.O. Box 56164

Madison,  WI  53705

(608) 274-1946

IBM is a registered trademark of

International Business Machines Corp.

Microsoft is a registered trademark of

Microsoft Corporation.

Hercules is a registered trademark of

Hercules Computer Technology, Inc.

Page 1 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Table of Contents

Disclaimer...................................  3

Acknowledgements.............................  3

General Information..........................  4

System Requirements..........................  4

Some Definitions.............................  4

Getting Started..............................  5

Viruses, Corrupt files and File Integrity....  5

Tips & Techniques............................  6

Maximums.....................................  6

Information about PCBRIDGE files.............  6

DOC files................................  6

DATA files...............................  7

PROGRAM files............................  7

Program details..............................  8

PCBRIDGE.................................  8

Input..................................  8

Lane Loads............................. 11

Maximum Negative Moment................ 12

External Vehicle Live Load Catalog..... 14

User-Cataloged Vehicles................ 14

Output................................. 15

Limitations............................ 15

Graphics Hardcopy............................ 16

Run-Time Errors.............................. 16

References................................... 16

PCBRIDGE Version / Enhancements.............. 17

Obtaining the Latest Version................. 18

Some Questions answered...................... 19

Permission to COPY........................... 20

User-Supported Software...................... 21

Shareware.................................... 23

Documenting Problems......................... 24

Invoice...................................... 24

8087 Version................................. 25

Command Line Switch/n........................ 25

Test Machines................................ 26

Index........................................ 27

Page 2 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Disclaimer

----------

Use  of  this  program  acknowledges  this   disclaimer   of

warranty:

This program is supplied as-is.  The  Author  disclaims  all

warranties,   expressed   or   implied,  including,  without

limitation, the warranties of merchantability and of fitness

of this program for any purpose.

Although every effort has been made to  ensure  both  theory

and  programming  logic  in  this  program  are correct,  no

warranty, either expressed or implied is made by the  Author

that the analyses obtained from its use will be correct.  In

no  event  will the Author be liable to you for any damages,

including any lost profits, lost savings or other incidental

or consequential damages  arising  out  of  the  use  of  or

inability  to  use this program, even if the Author has been

advised of the possibility of such damages, or for any claim

by any other party.

Acknowledgement

---------------

I gratefully acknowledge the following engineers

Michael Ritter, P.E.

Ross Lanius, Jr., P.E.

They and users are entitled to credit for things you like in

the program.

Page 3 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

General Information

-------------------

PCBRIDGE analyzes the  forces  on  a  continuous  beam  over

simple  supports.   Each  span can have a different flexural

stiffness EI.  A series of concentrated loads  is  "stepped"

along  the  beam and maximum moment (positive and negative),

shear (absolute), and deflection (negative) are  stored  for

presentation as load ENVELOPES.  PCBRIDGE can also analyze a

single span bridge and/or stationary load(s).

PCBRIDGE employs a matrix  method  of  structural  analysis,

specifically  the three-moment method of solution along with

singularity  functions  to  calculate  shear,   moment   and

deflection at any location along the bridge.  See section on

References.

For more information see section on Program details.

System Requirements

-------------------

PCBRIDGE requires:

1) structural engineering knowledge

(most IMPORTANT)

2) IBM PC or IBM XT or IBM AT with:

a) DOS 2.10 or higher

b) 256K memory

c) 1 360K disk drive

d) CGA, EGA, VGA, MCGA, HGC, or HGC+

80 column monitor

Some Definitions

----------------

Consider the bridge starts on  the  left  and  ends  on  the

right.   The  distance is measured from the left.  Spans are

numbered again from left to right.

Axle loads are positioned front to back of the vehicle.

Applied loads are positive downward.

At each distance on the bridge the position of  the  vehicle

causing  the  maximum  moment  (or  shear  or deflection) is

calculated.

Page 4 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

For example:

At bridge position 35.5 (can be span 2 position 15.5)

maximum moment of 67.4 occurs when the vehicle

is at bridge position 40.0L .

40.0L means the front axle of the vehicle is at 40.0

and the L means the vehicle is facing Left.

The Deflect(ion) Coeff(icient) is EIv/S  where  EI  is  span

flexural  rigidity,  v  is  the deflection and S is the Span

length.  Multiply the deflection coefficient value  by  S/EI

to get v or divide EI by the deflection coefficient value to

get S/v .

Getting Started

---------------

First make a copy of the PCBRIDGE disk.

1. Boot up the computer.   Put  the  PCBRIDGE  Disk  in  the

drive.

2. Type PCBRIDGE (e.g. A>pcbridge)

3. You will be  queried  for  the  type  of  vehicle.   Give

appropriate response.

4. After you make your choice, you will be asked for  bridge

data,   output  file  (or  project) information.  Again give

appropriate responses.

5. That's all there is to it.

Viruses, Corrupt files and File Integrity

-----------------------------------------

EFILECHK checks if even one byte has  been  changed  in  the

.EXE   files   developed  and  distributed  by  the  author.

EFILECHK checks the .EXE files  in  the  default  directory.

You should get IBM extended character 251 (square root mark)

if  the  .EXE  file  is  OK.   If  you  get  an IBM extended

character 168 (upside down question mark), call  Joe  Murphy

immediately.

Page 5 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Tips & Techniques

-----------------

Keep units CONSISTENT!

All length units must be the same.

All force (weight) units must be the same.

To get quickly from one field to another several fields away

(e.g. there are 76  input  fields)  just  HOLD  the  up/down

cursor keys down. The key are typematic.

Maximums

--------

PCBRIDGE has the following maximums:

Number of spans                    =    10

Number of concentrated loads       =    16

Total bridge length                =  1020

Information about the files on the PCBRIDGE Disk

------------------------------------------------

This  information  is  provided  so  that  you'll  know  the

function of each file distributed to you or made by

PCBRIDGE,  and  whether  or  not each file is optional.  The

following files have a "+" if provided on  the  distribution

diskettes and have a "*" if optional:

DOC files

---------

+*  ADDED.DOC  -  If  this  file  is  present,  it  contains

additional documentation.

+*  DOC.BAT - The file is used to assist you in printing out

the  documentation.   It  is  also  not  needed  after   the

documentation has been printed.

+*  INVOICE.DOC - A one-page ASCII  text  file  form  to  be

filled out and sent along with registration fee.

+*  PCBRIDGE.DOC - This is the main documentation file.   It

is not needed after the documentation has been printed.

Page 6 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

+*  PROBLEM.DOC - A ASCII text file form to  be  filled  out

and  sent  along with any INPUT files,  if the author cannot

resolve the  problem  by  phone.   Also  use  the  form  for

documentation errors or enhancement suggestions.

+*   README1.DOC  -  A  file on steps to use PCBRIDGE.  This

file is not needed after the documentation has been printed.

DATA files

----------

+*  FACTOR.IMP - ASCII file containing numbers to define the

IMPact  FACTOR  for  live  loads.  (See  section  on  Impact

Factor).

+*  CATALOG8.VEH - Example user-defined catalog of 8 vehicle

types. (See section on External Vehicle Live Load Catalog).

+*  #.VEH - Example user-cataloged vehicle. (See section  on

User-Cataloged Vehicles).

PROGRAM files

-------------

+*  EFILECHK.EXE - A stand-alone  program  that  checks  the

file   integrity  of  all  the  *.EXE  files  developed  and

distributed by the author.  See Section on Viruses,  Corrupt

Files and File Integrity.

+*   HIRESPIX.EXE  -  A   stand-alone,   terminate-and-stay-

resident  (TSR),  program  similar to IBM's GRAPHICS.COM for

Epson and compatible dot matrix printers.  Works  with  CGA,

EGA, VGA, MCGA, HGC and HGC+.

+*  MSHERC.COM - A stand-alone,  terminate-and-stay-resident

(TSR), driver for monographics with Hercules HGC and HGC+.

+    PCBRIDGE.EXE  - A program that does the continuous beam

force  analysis,  storing the  maximum  moments,  shear  and

deflection.

+*  README1.EXE - A program to display README1.DOC.

+*  VIDFIX.EXE - A stand-alone,  terminate-and-stay-resident

(TSR),  patch for early-IBM BIOS.  If PCBRIDGE seems to lose

its way and  you  have  an  early-IBM,   run  VIDFIX  before

running PCBRIDGE.

Page 7 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Program details

--------------

PCBRIDGE

--------

PCBRIDGE uses the Three-moment method to  form  a  symmetric

tri-diagonal  matrix  and solves for the internal moments at

the interior supports of the bridge.  After the moments  are

obtained   the   support  reactions  are  determined.   With

reactions and moments at the ends of simply supported  beams

as  knowns,  singularity  functions are used to find moment,

shear and deflection at any location.   If  the  values  are

larger than previously stored values they are swapped.

This process is then repeated as the  vehicle  is  "stepped"

0.5  unit  forward.   Again  and  again until the vehicle is

completely across  the  bridge.   On  the  second  pass  the

vehicle is "backed" left to right across the bridge.

On very large bridges (600.0) with 10 spans the program  can

take  some  time to run (dependent on IBM model, etc).  This

maximum  case  had  a  L90  vehicle  and  had  1201  storage

locations with over 2430 steps.

Input

-----

Live Load Selection Screen

This screen allows the selection  of  a  cataloged  vehicle.

Selection of a specific vehicle from an external or internal

catalog  automatically inputs the appropriate axle loads and

axle spacings.  The automatic input can still be  edited  or

modified but not stored by PCBRIDGE.  To use up to 10 "user-

cataloged" vehicles see section on User-Cataloged Vehicles.

If the file "CATALOG8.VEH"  cannot be  found  then  PCBRIDGE

uses  its'  internal  catalog.   (See  section  on  External

Vehicle Live Load Catalog.)

Page 8 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

PCBRIDGE's internal vehicle catalog contains 8 commonly used

vehicle types. (User can create custom vehicle catalog).

The vehicle axle loads are in kips.

Axle spacings are in feet.

Vehicles HS 20-44, HS 15-44, U80, and U120 have variable

spacing between axles but those in the internal catalog have

the minimal spacing.

L90 tracked vehicle  is  approximated  as  a  series  of

concentrated  loads in kips spaced at .5 foot intervals over

a total distance of 15 feet for a total of 180 kips.

Data Input Screen

This screen has 76 input fields for data on the  bridge  and

vehicle.

Assistance in input can be obtained by pressing the F1  key.

Program  calculation  continues by pressing the [Esc] key at

any point in the data.

The cursor can "fly" forward or backward between data fields

by holding down the Up or Down arrow keys.  Where a  default

value  is  desired  no  action  is  required,  otherwise the

default value should be typed over and/or deleted.

Span Input

a) Span (Number) - Left to right numbering

b) (Span) Length - specify to nearest .5 unit

(keep units consistent)

c) EI(rel) - Dimensionless relative EI of each span

in relation to the other spans - Default 1

d) Dead Load - uniformly distributed load on each span-

force/length - Default 0

e) LL - Lane Load Flags (0/1) for discontinuous

or continuous lane loading - Default 0

File for Output

Field requires appropriate filename so generated

output can be printed to disk file.  A full filename

would include a letter drive, colon, 8 character

filename, period, and a three character extension

(i.e.  D:PROJECT3.BDG ). Default "NUL"

Page 9 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Load Distribution Factor

This is a factor applied to live concentrated loads.

A factor of 0.50 might be applied to get WHEEL loads

from AXLE loads.  Default 1

Position of vehicle

A value of 0 calculates the envelopes for a moving

vehicle both left-to-right and back.  The absolute

value of a non-zero entry positions the front axle

of the stationary vehicle. A negative value has the

vehicle face left. Default 0

Impact Factor

Y specifies to include the impact factor for each

span (Default N). This factor INCREASES live load

values. (Applies to concentrated AND lane loads!)

If the file "FACTOR.IMP" cannot be found and Y is

specified,  PCBRIDGE assumes the AASHTO impact

factor defined below:

50

factor for impact =  1 + _________________  <  1.3

125 + Span Length

This factor requires span length in feet.

If the file "FACTOR.IMP" is found in the default

directory and Y is specified,  PCBRIDGE reads three

values from the file A,B,C and computes an impact

factor according to:

A

factor for impact =  1 + _________________  <  C

B  + Span Length

In this case, the units of span length have to be

consistent with A and B and not necessarily in feet!

Page 10 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Vehicle Data

Axle Number - Front to back numbering

Axle Spacing - Distance from previous axle or

concentrated load - length units.

Axle Load - Load corresponding to axle number

POSITIVE value means load is acting

down.

Lane Loads

----------

Option 91

To have a moving concentrated load  with  continuous  or

discontinuous lane loads the user must input the following:

1) Select "User defined" at Vehicle Live Loads Screen

2) In the Span section LL: Change "0" to "1" to include

lane loading for appropriate spans

3) Set 0 for envelope data

4) Set axle #1 spacing to 0

Set axle #1 to concentrated force value

Set axle #2 spacing to 91

Set axle #2 spacing to lane load in force/length

NOTE: The uniformly distributed lane  load  entered  in  the

vehicle data is MULTIPLIED by the Impact Factor and ADDED to

the span dead load entered in the bridge span section.

The   concentrated   force   is   MULTIPLIED   by  the  Load

Distribution Factor and then MULTIPLIED by the Impact Factor

corresponding to the span it is on.

Page 11 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Maximum Negative Moment

-----------------------

Option 92

To help in calculating maximum negative moments PCBRIDGE

utilizes two independently moving  concentrated  loads  with

continuous  or discontinuous lane loads. The user must input

the following:

1) Select "User defined" at Vehicle Live Loads Screen

2) In the Span section LL: Change "0" to "1" to include

lane loading for appropriate spans

3) Set 0 for envelope data

4) Set axle #1 spacing to 0

Set axle #1 to concentrated force value

Set axle #2 spacing to 92

Set axle #2 spacing to lane load in force/length

NOTE: The uniformly distributed lane  load  entered  in  the

vehicle data is MULTIPLIED by the Impact Factor and ADDED to

the span dead load entered in the bridge span section.

The   concentrated   forces   are  MULTIPLIED  by  the  Load

Distribution Factor and  then  each  is  MULTIPLIED  by  the

Impact Factor corresponding to the span it is on.

These two concentrated loads traverse the entire bridge with

the only requirement that they are  not  on  the  same  span

simultaneously.

Option 93

If in step 4) the user Sets axle spacing #2 to 93, then  the

two concentrated loads are also required to stay in adjacent

or adjoining spans.

Page 12 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Option 94

To check for ALL POSSIBLE combinations of continuous and

discontinuous  lane  loads,  the  user  should   input   the

following:

1) Select "User defined" at Vehicle Live Loads Screen

2) In the File for Output section, specify a legitimate

filename, DO NOT USE "NUL"

3) Set 0 for envelope data

4) Set axle #1 spacing to 0

Set axle #1 to concentrated force value

Set axle #2 spacing to 94

Set axle #2 spacing to lane load in force/length

Two concentrated loads traverse the entire bridge  with  the

only  requirement  that  they  are  not  on  the  same  span

simultaneously.   This  is  repeated  for   EVERY   possible

combination of lane loads included AND not included for each

span.   Obviously  this  makes for 2-to-the-power-M combina-

tions, where M = number of spans.  This can run  into  hours

of  computational time!  That is why an output filename MUST

be specified.

Option 95

If in step 4) the user Sets axle spacing #2 to 95, then  the

two concentrated loads are also required to stay in adjacent

or adjoining spans.

Option 96

This option is the same as Option 94 but  only  the  support

values are checked or stored.  Axle spacing #2 is 96.

Option 97

This option is the same as Option 95 but  only  the  support

values are checked or stored.  Axle spacing #2 is 97.

Page 13 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

External Vehicle Live Load Catalog

----------------------------------

If the file  "CATALOG8.VEH"  is  found  in  the  default

directory,   PCBRIDGE  reads  8 lines describing 8 vehicles.

There MUST be 8 vehicles.  This ASCII file has  8  lines  in

it, each in the form:

"8char_ID",NA,0,L1,S12,L2,S23, ... ,L(N-1),S(N-1)N,LN

See next section for field definitions.

User-Cataloged Vehicles

-----------------------

Users can "catalog" up to 10 vehicles.  Each vehicle has

to be in a ASCII file with  filenames  of  0.VEH  to  9.VEH.

When  in  the  live load selection menu, pressing the number

keys 0 through 9 will have PCBRIDGE select  your  previously

cataloged  vehicle.  To define a user-cataloged vehicle, the

user has to use a word processor or line editor like  EDLIN.

The ASCII file has 1 line in it of the form:

"8char_ID",NA,0,L1,S12,L2,S23, ... ,L(N-1),S(N-1)N,LN

where the  first  field  in  quotes  is  an  8  alphanumeric

character ID

the second field is the number of axles

the third field is 0

the fourth field is axle load 1

the fifth field is spacing between axle 1 and 2

:

:

up to the load on the "n"th axle

As an example:

"HS 20-44",3,0,8,14,32,14,32

would duplicate selection a)  on  the  live  load  selection

menu.

Page 14 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Output

------

Input values are repeated in the output.

The date, time, and time of analysis is also recorded.

At every 0.5 units of the  bridge,  values  of  maximum:

+moment,  -moment,  absolute  value of shear and -deflection

(coefficient); are given for  moving  vehicle  loads  (hence

envelopes).  Also given is the position of the front axle of

the vehicle on the bridge which causes the maximum to occur.

An "L" means the vehicle is facing (and moving) Left.

If the  vehicle  is  stationary,  shear  and  deflection

(coefficient) have the correct sign.

If one does a Maximum Negative Moment analysis,  in  the

output  the -moment is given along with the positions of the

two concentrated loads that cause the maximum.  For  options

94,95,96,97  the  lane  load  flags  are also included which

produce the maximum negative moment.

Limitations

-----------

Use of this program is subject to the following limitations:

1. Maximum number of spans is 10.

2. Maximum number of concentrated loads is 16.

3. All supports are assumed simple (hinge or roller).

4. Beams must be continuous over all interior supports.

5. Lengths and distances must be to the nearest .5 unit.

(The program rounds to the nearest .5 unit)

6. Maximum total bridge length is 1020.0

7. The flexural rigidity EI for any individual span is

assumed constant over the entire span length.  When

different values are specified for adjacent spans,

the transition is assumed to occur at the support

centerline.

Page 15 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Graphics Hardcopy

-----------------

Using DOS 2.10 or later and an IBM dot  matrix  printer  (or

one that will understand the graphics commands) you can do a

graphics screen print. Before running PCBRIDGE type GRAPHICS

while  in  DOS command (e.g. A>graphics ). Then while in the

graphics of PCBRIDGE you can print the  screen  by  pressing

the  Shift-PrtSc  keys.   Refer  to your DOS manual GRAPHICS

Command.

HIRESPIX is provided and is similar to  IBM's  GRAPHICS  but

prints  a  smaller  graph.   Install HIRESPIX instead of, or

AFTER, GRAPHICS.

HIRESPIX works with CGA, EGA, VGA, MCGA, HGC and HGC+ and an

IBM dot matrix printer or compatible.

Run-Time Errors

---------------

To reduce the size of the .EXE modules QuickBASIC's ON ERROR

GOTO was eliminated.  Check to make  sure  drive  doors  are

closed,  printer is turned on,  etc.  The program does check

for input files existing, as well as,  87  math  coprocessor

exception    conditions   (invalid   operation,   underflow,

overflow, divide by zero, etc.)

References

----------

Norris, C.H., and J.B. Wilbur

1960.  Elementary Structural Analysis.

McGraw-Hill Book Company,

New York.

Popov, E.P.

1968.  Introduction to Mechanics of Solids.

Prentice-Hall, Inc.,

Englewood Cliffs.

Page 16 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

PCBRIDGE Versions / Enhancements

--------------------------------

1.0  -  First documented distributed program

-  10 spans, 16 loads, moving load envelopes

1.5  -  include individual span dead load

-  include lane loading

-  change input notation

POSITIVE loads act DOWNWARD

1.6  -  fix some single span problems

1.87 -  fix dead load problem

-  include user-cataloged vehicles (up to 10)

-  requires 87 math coprocessing chip

2.00 -  autocheck and autoutilization of 87 math

coprocessing chip

2.02 -  command line override switch for 87 math

coprocessing chip

2.04 -  fix reaction reinitialization

-  does NOT multiply the span dead load

by the AASHTO impact factor.

-  DOES multiply a concentrated load over a support

by the larger of the two impact factors

(from the spans on either side)

2.10 -  compiled with QuickBASIC 4.0

(runs faster, larger program)

-  command line override switch for form feeds

2.50 -  summary analysis, list of error codes

2.60 -  maximum bridge length increased to 600

-  vehicle steps are echoed to the screen

-  graphics and HiResPix (screen dump driver)

works with CGA, EGA, VGA, MCGA, HGC and HGC+

and 100% compatibles

-  user-defined impact factor using ASCII file

named FACTOR.IMP

-  major revision of documentation with INDEX

-  separate INVOICE.DOC and PROBLEM.DOC

-  EFILECHK for .EXE file integrity check

-  VIDFIX.EXE for early-IBM video bug

-  checks 87 math coprocessor for exception

conditions (i.e. errors)

Page 17 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

2.80 -  maximum bridge length increased to 1020

-  user-defined external vehicle live load

catalog using ASCII file named CATALOG8.VEH

-  lane load flags for continuous and

discontinuous lane loadings

-  two independently moving concentrated

loads with lane loads to produce

maximum negative moment (6 options)

Obtaining the Latest Version

----------------------------

The latest version of PCBRIDGE will be posted  on  CIME-ISE

in  the file BRDG28#.ZIP.   The # is a minor revision number

0 through 9.   I  will  notify  registered  users  of  major

revisions.

CIME-ISE is the electronic bulletin board system  (BBS)  of

the  American  Society of Mechanical Engineers.  The acronym

stands for Computers In Mechanical Engineering - Information

and  Software  Exchange.   CIME-ISE  is  on-line  24   hours

everyday.   The  system  answers 5 phone lines at (608) 233-

3378 using 300, 1200, 2400,  or  9600  baud.   The  CIME-ISE

library currently has more than 4500 programs of interest to

Mechanical  Engineers.   All  software can be freely copied.

There are no usage or registration fees involved and  anyone

interested in technical or scientific software is invited to

call.

Page 18 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Some Questions answered

-----------------------

1.  "In what language was PCBRIDGE  written?".   ANSWER:  It

was  written  in  BASIC, then compiled with a QuickBASIC 4.5

Compiler.

2.  "Can  I  obtain  the  source  code  for  the  program?".

ANSWER: No.

3.  "What is your update policy?".  ANSWER: Presently I will

notify registered users of updates by postcard.  Or you  can

mail  me  a diskette with postage and mailer, or a check for

$10, every six to nine months or so, and I will send you the

then-current version.

4.   "Do  you  have  any  other  programs   that   you   are

distributing?".  ANSWER:  Yes,  PC-STRAN(tm) is a STRuctural

ANalysis program for member structures.

5.  "Is there a version of PCBRIDGE  for  other  computers?"

ANSWER: No.

6.  "What about  program  support?".  ANSWER:  I  will  give

support  and reply to written questions of registered users.

I will also notify registered users of any program problems.

Page 19 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Permission to COPY

------------------

Clubs, other non-profit organizations and  vendors  approved

by  the  Association  of  Shareware  Professionals (ASP) are

granted permission  by  the  author  to  freely  copy  these

programs  and  documentation and share it with their members

and customers, so long as:

1.  No price is charged for the software  or  documentation.

However,  a distribution cost may be charged for the cost of

the diskettes, so long  as  it  is  not  more  than  $5  per

diskette.

2.  Club  members  and  ASP-approved  vendor  customers  are

informed  of  the  user-supported  concept and encouraged to

support it with their registrations.

3.  The program or documentation are not modified in any way

and are distributed together.

[NOTE: If you receive the software  through  a  club,  other

non-profit  organization,  friend or an ASP-approved vendor,

you have NOT registered the software with the  author!   You

MUST  register  the  software with the author by sending the

registration fee to the author!]

See User-Supported Software.

Page 20 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

User-Supported Software

-----------------------

User-Supported Software

If you are using this program

your registration, $35, is required.

Joe Murphy

Post Office Box 56164

Madison,  WI  53705

You are encouraged to copy and share these programs.

________tm

PCBRIDGE   is NOT public domain or free  software,   but  is

being distributed as "user-supported" software.

Non-registered users of this software are granted a  limited

license  to  make  an  evaluation  copy  for  trial use on a

private, non-commercial basis,  for the express  purpose  of

determining  whether  the  programs  are  suitable for their

needs.  At the end of this trial period, you  should  either

register your copy or discontinue using the programs.

This means that if you use this program for any real benefit

to yourself or others, you should pay for your  copy.   That

way  the author will be able to provide you with support and

updates.

A registration entitles you to use the program  on  any  and

all   computers   available  to  you,   with  the  following

restriction:  If others have access to this program  or  may

use it,  then you should purchase a site license.

Page 21 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

All users are granted a limited license to copy the programs

for  the  trial  use  of others and subject to the following

limitations.  This license does NOT include distribution  or

copying of this software package:

1. In connection with any other product or service.

2. For general use within a company or institution.

3. For any consideration fee.

4. Distribution in modified form,

i.e., the documentation must be included.

User-supported software is a way  in  distributing  computer

programs, based on these beliefs:

1.  That the value and utility of software is best  assessed

by the user on his/her own system.

2.  That the creation of personal computer software can  and

should be supported by the computing community.

3.  That distribution  of  programs  should  be  encouraged,

rather than restricted.

Anyone may request a copy of  a  user-supported  program  by

sending  blank,  formatted  disk(s)  to  the  author  of the

program.  An  addressed,  postage-paid  return  mailer  must

accompany  the  disk  (no exceptions, please). A copy of the

program, with documentation on the disk,  will  be  sent  by

return  mail.   The  program  will  carry  a notice with the

registration fee and the program's  author.   A  person  who

uses the program is expected to become a registered user.

Free distribution of software and registration for  its  use

eliminates   costs   for  advertising  and  copy  protection

schemes.

Users obtain quality software at a  reasonable  price.  They

can  try  it  out before buying, and do so at their own pace

and in the comfort of their own home or office.

User-supported  software  only  works  if  the  software  is

actually  user  supported.   If  you  use  the software, you

should show your support and register your copy.  It's  only

through  your  registration  that  the  author can afford to

improve the software and bring you  improved  versions.   If

you  don't  like  the  software,   or feel that it is sorely

lacking in some area(s),  then please don't hesitate to  let

the author know.

Page 22 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Shareware

---------

Shareware is a unique method of marketing  software  based

on the try-before-you-buy concept.  The authors of shareware

programs  retain all rights to the software (copyright), and

they require a "license fee" from those persons who continue

to use the software.  The term shareware goes  back  to  the

early  1980's  when  there was a column in Infoworld by that

name  describing  programs  available   through   electronic

Bulletin  Board Systems.  In 1984, Nelson Ford had a contest

in his Softalk column on user-supported  and  public  domain

software.   The purpose of this contest was to find a better

name than "user-supported" for  this  software  genre.   The

overwhelming   favorite   was  shareware  and  he  announced

shareware the winner.  Since then, the name has increased in

use by the common public.  Shareware is a concept  that  has

wide  respect  among  part  of  the user community who never

doubted that it is a generic term.

The   Association   of   Shareware    Professionals    was

incorporated  in  August,  1987, to strengthen the future of

shareware software as an alternative to  marketed  software.

This   organization   consists   of  shareware  authors  and

associate  vendor  members  nationwide  who  desire  to  see

recognized standards for quality software and documentation.

For  more  information,  write to:  Association of Shareware

Professionals,  Post Office Box 5786, Bellevue, WA  98006.

_______

____|__     |               (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

Joe Murphy is a member of  the  Association  of  Shareware

Professionals  (ASP).   ASP  wants  to  make  sure  that the

shareware principle works for you.  If  you  are  unable  to

resolve  a  shareware  related problem with an ASP member by

contacting the member directly,  ASP may be  able  to  help.

The  ASP Ombudsman can help you resolve a dispute or problem

with an ASP member, but does not provide  technical  support

for members' products.  Please write to the ASP Ombudsman at

P.O.  Box  5786,  Bellevue,  WA  98006  or send a Compuserve

message via easyplex to ASP Ombudsman 70007,3536 .

Page 23 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Documenting Problems

--------------------

If in using this shareware software you encounter a problem,

(software   bugs,   documentation   errors,   or   suggested

enhancements)  and  after  rereading  the  documentation and

after calling the author for help,   send  DOCumentation  of

the  problem  with input files (on disk) to the author along

with the filled out  form  in  PROBLEM.DOC.   This  way  the

author can try to resolve the problem.

Invoice

-------

If you have need for an invoice so  that  you  can  initiate

payment  through  your company accounting system, an invoice

is provided in  INVOICE.DOC.   For  those  of  you  in  this

category,  or  for  those  who  wish an invoice for your tax

records, it should serve the purpose.

Page 24 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

8087 Version

------------

PCBRIDGE v2.00 and later checks if a  87  math  coprocessing

chip  is  available.   If  PCBRIDGE  detects it is, PCBRIDGE

branches to code that utilize assembly language  subroutines

written for the 87 chip.

Command Line Switches/n

-----------------------

________tm

PCBRIDGE   has an override switch at the command  line.   To

use the override option, type at the command line

pcbridge/n

n     [form feed]      [87 in PC clone]

1         no

2                            no

3         no                 no

[form feed]

The program inserts a form feed at the  start  and  end  of

printing.  If  you do not want these form feeds inserted use

option 1 or 3.

[87 in PC clone]

On some PC clones the program senses  an  87  chip  present

when one is not installed.  If this is the case use option 2

or 3.

Page 25 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Test Machines

-------------

PCBRIDGE version 2.80 has been developed and tested  on  the

following machines:

IBM PC1 64K motherboard, upgraded BIOS

CGA monitor

V20 chip, 4.77MHz 8087

PC DOS 3.00

ALR 386/220

VGA monitor, Hercules Graphics Card Plus

and monochrome monitor

80386 and 80387 20MHz

PC DOS 3.30

DATAVUE Snap 1+1

V20 and 8087 4.77/9.54MHz

MS DOS 3.30

IBM PS/2 Model 25 (MCGA graphics)

8086 and 8087 8MHz

PC DOS 4.00

Page 26 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

Index

-----

AASHTO, 10, 17

absolute, 4, 10, 15

ADDED.DOC, 6

alphanumeric, 14

answer, 2, 18, 19

approximate, 9

ASCII, 6, 7, 14, 17, 18

ASP, 20, 23

assembly, 25

autocheck, 17

automatically, 8

autoutilization, 17

axle, 4, 5, 8, 9, 10, 11, 12, 13, 14, 15

backward, 9

BBS, 18

beam, 1, 4, 7, 8, 15

boot, 5

BRDG28#.ZIP, 18

bridge, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15,

16, 17, 18, 19, 20,

21, 22, 23, 24, 25, 26

bug, 17, 24

catalog, 2, 7, 8, 9, 14, 17, 18

CATALOG8.VEH, 7, 8, 14, 18

centerline, 15

CGA, 4, 7, 16, 17, 26

CIME-ISE, 18

clone, 25

coefficient, 5, 15

column, 4, 23

compatible, 7, 16, 17

compuserve, 23

concentrated, 4, 6, 9, 10, 11, 12, 13, 15, 17, 18

concept, 20, 23

conditions, 16, 17

connection, 22

consistent, 6, 9, 10

constant, 15

continuous, 1, 4, 7, 9, 11, 12, 13, 15, 18

coprocessor, 16, 17

copyright, 1, 23

corrupt, 2, 5, 7

Page 27 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

dead, 9, 11, 12, 17

default, 5, 9, 10, 14

deflection, 4, 5, 7, 8, 15

details, 2, 4, 8

dimensionless, 9

disclaimer, 2, 3

dispute, 23

distributed, 5, 6, 7, 9, 11, 12, 17, 20, 21

DOC.BAT, 6

documentation, 6, 7, 17, 20, 22, 23, 24

DOS, 4, 16, 26

dump, 17

edited, 8

EFILECHK.EXE, 7

EGA, 2, 4, 7, 10, 12, 15, 16, 17, 18

EI, 3, 4, 5, 6, 9, 15, 17, 20, 21, 22

EIv/S, 5

electronic, 18, 23

engineer, 3, 4, 18

enhancement, 2, 7, 17, 24

envelopes, 4, 10, 15, 17

error, 2, 7, 16, 17, 24

example, 5, 7, 14

exception, 16, 17, 22

facing, 5, 15

factor, 7, 10, 11, 12, 17

FACTOR.IMP, 7, 10, 17

fields, 6, 9

filename, 9, 13, 14

filenames, 14

flexural, 4, 5, 15

force, 4, 6, 7, 9, 11, 12, 13

force/length, 9, 11, 12, 13

free, 18, 20, 21, 22

function, 4, 6, 8

GRAPHICS.COM, 7

hardcopy, 2, 16

Hercules, 1, 7, 26

HGC, 4, 7, 16, 17

hinge, 15

HIRESPIX.EXE, 7

impact, 7, 10, 11, 12, 17

input, 2, 6, 7, 8, 9, 11, 12, 13, 15, 16, 17, 24

install, 16, 25

INVOICE.DOC, 6, 17, 24

Page 28 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

knowledge, 2, 3, 4

left-to-right, 10

license, 21, 22, 23

limitation, 2, 3, 15, 22

live, 2, 7, 8, 10, 11, 12, 13, 14, 18

load, 1, 2, 4, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 17, 18

location, 4, 8

logic, 3

mail, 19, 22

maximum, 2, 4, 5, 6, 7, 8, 12, 15, 17, 18

maximum negative moment, 2, 12, 15, 18

MCGA, 4, 7, 16, 17, 26

member, 19, 20, 23

module, 16

moment, 2, 4, 5, 7, 8, 12, 15, 18

monitor, 4, 26

monochrome, 26

monographics, 7

moving, 1, 10, 11, 12, 15, 17, 18

MSHERC.COM, 7

non-zero, 10

notation, 17

number, 4, 6, 7, 9, 11, 13, 14, 15, 18

ombudsman, 23

option, 6, 11, 12, 13, 15, 18, 25

output, 2, 5, 9, 13, 15

overflow, 16

override, 17, 25

pass, 8

payment, 24

PC-STRAN(tm), 19

PCBRIDGE.DOC, 6

PCBRIDGE.EXE, 7

permission, 2, 20

phone, 7, 18

policy, 19

position, 4, 5, 10, 15

positive, 4, 11, 17

PROBLEM.DOC, 7, 17, 24

processor, 14, 16, 17

project, 5, 9

protection, 22

purpose, 3, 21, 23, 24

Page 29 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

quality, 22, 23

question, 2, 5, 19

QuickBASIC, 16, 17, 19

reaction, 8, 17

README1.DOC, 7

README1.EXE, 7

record, 15, 24

references, 2, 4, 16

register, 1, 18, 19, 20, 21, 22

registered, 1, 18, 19, 20, 21, 22

registration, 6, 18, 20, 21, 22

relative, 9

repeated, 8, 13, 15

require, 2, 4, 9, 10, 12, 13, 17, 21, 23

restriction, 21

revision, 17, 18

rights, 23

rigidity, 5, 15

run-time, 2, 16

S/EI, 5

S/v, 5

section, 4, 7, 8, 11, 12, 13, 14

Shareware, 2, 20, 23, 24

shear, 4, 7, 8, 15

singular, 4, 8

site, 21

solution, 4

solve, 7, 8, 23, 24

source, 19

span, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 17

specific, 4, 8

stand-alone, 7

standards, 23

stationary, 4, 10, 15

step, 4, 7, 8, 12, 13, 17

stiffness, 4

storage, 8

structural, 4, 16, 19

structure, 19

suggestions, 7

support, 2, 4, 8, 13, 15, 17, 19, 20, 21, 22, 23

switches, 25

system, 2, 4, 18, 22, 23, 24

technical, 18, 23

techniques, 2, 6

terminate-and-stay-resident, 7

theory, 3

Page 30 of 31

________tm

PCBRIDGE    v2.8x  Users Guide

three-moment, 4, 8

tips, 2, 6

transition, 15

TSR, 7

types, 7, 9

underflow, 16

unit, 6, 8, 9, 10, 11, 15, 22, 23

units, 6, 9, 10, 11, 15

update, 19, 21

upgrade, 26

user-cataloged, 2, 7, 8, 14, 17

user-defined, 7, 17, 18

user-supported, 2, 20, 21, 22, 23

.VEH, 7, 8, 14, 18

vehicle, 2, 4, 5, 7, 8, 9, 10, 11, 12, 13, 14, 15, 17, 18

VGA, 4, 7, 16, 17, 26

video, 17

VIDFIX.EXE, 7, 17

viruses, 2, 5, 7

warranty, 3

wheel, 10

write, 23

zero, 10, 16

Page 31 of 31